1.0 Methods

1.1 Aircraft measurements

Table S1 provides a summary of the measurements with associated instrumentation, technical details and related references.

1.1.1 Trace gas measurements

All of the trace gas instrumentation except NH3 and the CIMS were sampled through PTFE tubing from a main aircraft roof hatch that contained multiple inlet ports through which rear-facing tubing was mounted.  The rear-facing inlet minimized the sampling of particles.

NO, NO2, NOy, SO2  NO and SO2 were directly measured: NO by chemiluminescence with excess ozone using a 42i TL instrument operated in single mode, while SO2 was measured by pulsed fluorescence with a 43i TL instrument.  A photolytic converter (Air Quality Design Inc.) was used to selectively convert a large fraction of NO2 to NO.  The sum of this NO2 fraction that was converted to NO plus ambient NO, defined as NOc, was measured by a second 42i TL chemiluminescent instrument and then NO2 was calculated based on NOc, NO and the efficiency of the photolytic converter.  NOy was measured by using an external molybdenum converter heated at 325 °C and placed as close as possible to the sampling point, followed by a third 42i TL instrument.  NO and SO2 calibrations were conducted by generating mixing ratios of 0-100 ppbv using NIST certified cylinders from Scott-Marin (10.3 ppmv accuracy: +/- 2 %), an Environics (Model 6100 Multi-Gas Calibrator), and a Sabio Zero Generator (Model 1001).  The efficiencies of the photolytic and NOy converters were determined using the gas phase titration option of the Environics calibrator.   Calibrations were conducted periodically throughout the measurement study using the NIST–certified standards.  Instrument zeros were performed for these instruments 3-5 times per flight for a duration of ~3-5 minutes each time at the beginning, during and after each flight.

NH3  Ammonia (NH3) was sampled through an unfiltered inlet, critical orifice and 4 m of 6.35 mm (¼”) outer diameter Sulfinert-coated tubing heated to 90 oC to minimize NH3 losses to the tubing walls.  The flow rate was 2.5 LPM, controlled through a critical orifice near the inlet with the pressure in the fluoropel-coated LGR cell being maintained at 100 Torr.  For instrument zeros, ambient air was passed through a Teflon filter coated with citric acid.  Calibrations were performed using a certified ammonia standard (Air Liquide; 10.0 ppm NH3 in N2, accuracy: +/- 5 %), diluted to near-ambient levels.

CO, CO2, CH4   CO, CO2, CH4 were measured with a Cavity Ring Down (CRD) spectroscopy instrument (Picarro G2401-m).

TC and NMOGT   A second Picarro G2401-m instrument was used to measure total carbon (TC, in units of ppmv C) by passing the sample air through a heated (650 °C) platinum catalyst (Shimadzu), adapted from Stockwell et al. (2018) and Veres et al., (2010) which converted all carbon species to CO2.  Approximately 4 g of platinum catalyst (https://www.elementalmicroanalysis.com/product_details.php?product=B1605&description=High%20Sensitivity%20Catalyst%20630-00996) was enclosed in a resistively heated ½” O.D. x 12” long stainless steel tube.  As the catalyst assembly was mounted on the roof exterior to the aircraft, with no unheated portion of inlet, TC losses were expected to be negligible.  NMOGT mixing ratios in units of ppmv C were quantified by subtracting the ambient CH4, CO and CO2 measurements (instrument without the catalyst) from the TC measurements (Fig. S2).  Calibrations using two different mixing ratio standards of CO, CO2 and CH4, traceable to NOAA GMD standards, were performed for both Picarro instruments during flight (at the beginning and end) to assess instrument drift and sensitivity.  No significant drift was observed during each flight.  NMOGT was averaged to 10 sec (from the 2 sec native time resolution) to increase the signal to noise ratio.  The uncertainty of each instrument was assessed in flight by overflowing the inlet with a constant flow of calibration gas or an ultra pure nitrogen gas stripped of CO2 via two NaOH pellet traps in series (https://www.sigmaaldrich.com/CA/en/product/supelco/503215).  In both cases, this resulted in an uncertainty of approximately 60 ppbv C at the 3σ level for each TC channel (dominated by the precision of the CO2 measurement) (Fig. S2).  These uncertainties were added in quadrature resulting in a 3σ uncertainty of ±85 ppb C for NMOGT.  Laboratory experiments indicated that the conversion efficiency of ethane across the catalyst was ~100 %, which is expected to be the most challenging species to combust aside from methane, which is concurrently measured.  Additional laboratory experiments using a range of hydrocarbons (>C2) including aromatics also exhibited ~100 % conversion efficiency (Li et al., 2021; Li et al., 2019).  The catalyst material was changed after approximately every 5 flights to further ensure minimal changes in efficiency.

CIMS  The CIMS instrument sampled from an insulated rear-facing inlet (PFA, 3/8” OD, ¼” ID) at 7 LPM (0°C, 1 atm).  The instrument was operated using iodide as a reagent ion.  The mass resolution at an internal standard peak (13CC2H6O2I-) was ~5400 Th/Th.  The reagent ion was generated by passing 2 sLpm (0°C, 1 atm) of UHP N2 over a methyl iodide permeation tube held at 40 °C.  This flow was then passed through a Polonium-210 ionizer (NRD P-2031) into the ion molecule reactor (IMR).  A flow of humidified N2 (20 sccm through a stainless steel bubbler) was also added to the IMR in order to keep the ratio of I(H2O)-/I- as constant as possible.  The IMR and small segmented quadrupoles (SSQ) were pressure controlled to 70 and 1.5 mBar respectively using Alicat pressure controllers (PC-EXTSEN).  Instrument zeros were performed every 15 min by flooding the inlet with 10 sLpm (0°C, 1 atm) of air that had been passed through a Pt/Pd catalyst (CD Nova) heated to 350 °C followed by bicarbonate and charcoal scrubbers (United Filtration).  A flow (50 sccm) of isotopically labelled propanoic acid (13CC2H6O2) was constantly added to the inlet during the campaign to track instrument sensitivity.  Compounds were identified using known/expected sensitivities and available calibration standards (Tables S3).

PTRMS  The PTR-ToF-MS  was operated in a configuration described previously (Table S1).  Gases with a proton affinity greater than that of water were protonated in the drift tube.  The pressure and temperature of the drift tube region were maintained at a constant 2.15 mbar and 60 °C, respectively for an E/N of 141 Td.  The unit contained a catalytic converter heated to 350 °C with a continuous flow of ambient air at a flow rate of one litre per minute.  A permeation tube with 1,2,4-trichlorbenzene was placed at the inlet to improve the accuracy of the mass calibration for higher masses.  Instrumental backgrounds were performed in flight using a custom-built zero-air generating unit.  The data were processed using Tofware software (Tofwerk AG).  Calibrations were performed on the ground using gas standard mixtures from Ionicon, Apel-Reimer and Scott-Marrin for 20 compounds (Table S3).  Compound identifications for molecular formulas for the PTRMS and CIMS data were assigned based on a limited set of possibilities (particularly for the smaller compounds), known or expected compound sensitivities, and previously published laboratory work by Koss et al. (2018); this is more fully described in the SI Sect. 2.1.3.

AWAS  The Advanced Whole Air Sampler (AWAS) was used to collect ambient samples using 1.33-litre electropolished stainless steel canisters in rack-mounted arrays of 12-canister modules (Lerner et al., 2017, and references therein).  A metal bellows compressor (Senior Aerospace Metal Bellows, MB-158) was used to pressurize canisters to approximately 30 PSI over a period of approximately 20 to 30 sec.  Sample lines and manifold tubing were continually flushed with ambient air during the flights.  Sampling took place by activating module and pump system valving with custom Labview-based software operating a data logger interface (Labjack Corp., Model T7).  The samples were analysed between 5 and 9 days after the flight with an analytical system installed at the Fort McMurray International Airport.  The on-site analytical system consisted of a custom fabricated gas chromatograph (GC) system using cryogenic sample pre-concentration, 2-dimensional gas chromatography, Mass Spectrometric Detection (MS) and Flame Ionization Detection (FID).  Sample air was cryogenically trapped at -185 °C on a glass bead-filled trap, thus condensing/solidifying the hydrocarbons, and subsequently thermally desorbing them at 135 °C into the multi-column, multi-oven GC/MS/FID instrument.  Trapped sample air volumes were calculated by recording pressure differences in a volume-calibrated downstream vacuum vessel before and after sample trapping.  Duplicate analysis was carried out on one canister in each AWAS module.  The analytical separation of approximately 120 chemical species was carried out by use of a pre-column (SPB-1) where the initial separation of compounds according to boiling point occurred.  The low molecular weight compounds (C2 to C4) were then directed to two RTX-QS columns connected in series and quantified by a FID.  The higher molecular weight compound stream (C4 to C10) was subsequently split and simultaneously analysed by a second FID connected to an Aluminum Oxide/KCL column (C4 to C8) and by a quadrupole Mass Spectrometer (Agilent Technologies, 5977B) connected to an HP-1 column by means of a fused silica tubing restrictor (C7 to C10).  The precolumn and RTX-QS columns were mounted in the main oven of the gas chromatograph (Agilent Technologies 7890B) and thus were subject to one temperature program.  The AlOx/KCL and HP-1 analytical columns were each mounted in a separate temperature-controlled GC oven module (Agilent Technologies, LTM Series II) and operated with a different temperature program.  Detector peak areas were calibrated with primary gas standard mixtures in the ppbv concentration range obtained from Apel-Reimer Environmental Inc. (U.S.A.) and the National Physical Laboratory (UK).  Compound retention time drift and potential detector sensitivity changes were monitored and compensated for by means of daily analysis of a secondary standard gas.  The AWAS modules were cleaned by a custom-fabricated, automated cleaning system similar to that of Lerner et al. (2017).    Toluene, benzene and xylenes were not quantified because the method and columns used were not optimized for these compounds.  Analytical issues due to incomplete water vapour management in the sample gas stream resulted in retention time shifts and some peak broadening effects resulting in elevated uncertainties.  Uncertainties are estimated at ±25 % for C2 to C5 and C6 alkanes detected by FID, and ±40 % for C6 alkenes, and the C7 to C10 species detected by MS.

Cartridges  Integrated gas phase samples were collected using an automated adsorbent tube sampling assembly (i.e. cartridge) that was mounted in an under-wing pod (see Ditto et al., 2021).  Adsorbent tubes were packed with quartz wool, glass beads, Tenax TA, and Carbopack X (or “QBTX”), similar to those discussed in Sheu et al. (2018).  Samples were shipped to Yale University where offline analysis was conducted using thermal desorption (GERSTEL TD 3.5+) followed by gas chromatography (Agilent 7890B), atmospheric pressure chemical ionization, and high-resolution mass spectrometry (Agilent 6550) to speciate gas-phase organic compounds (Ditto et al., 2021, Sheu et al., 2018, Khare et al., 2019).  Ion abundances for CH, CHO1, and CHS1 species were converted to mass concentrations assuming average response factors that were calculated based on calibrations using the NIST Reference Gulf of Mexico 2779 Macondo Crude oil reference material following Khare et al. (2019), which accounts for variations in response and fragmentation between components of the complex mixture.  CHN1 species were not quantitatively converted to mass due to the lack of comprehensive and available authentic standards.   The reported emissions are subject to potential variations in sampling efficiency within the under-wing sampling pod across C10-C25 and, in the event of losses due to analyte breakthrough, would likely be considered lower limit estimates.  Prior breakthrough testing with QBTX adsorbent tubes and similar sampling conditions to those used in this study showed that analyte trapping efficiency in the same carbon number range was generally greater than 85 % (Sheu et al., 2018).  For CH, CHO1, and CHS1, each group of isomers at a given carbon number was categorized by molecular formula, according to their double bond equivalents (DBE) ranging from 0 to 15.  Emission ratios (to in-plume CO) were estimated for CH, CHO1 and CHS1 using observed concentrations for the C10-C25 species summed across DBEs.  Further discussion of these methods can be found in Ditto et al. (2021), including in the SI (i.e., Section S3).

1.1.2 Particle measurements

Particles were sampled from a forward-facing isokinetic stainless steel diffuser inlet (Droplet Measurement Technologies) that was positioned near the top of the fuselage forward of the engine on the starboard side.  Theoretical calculations that take into account the inlet dimensions, volume flow and velocity indicated a 97 % transmission efficiency for particles < 1 µm through the inlet.  Air was pulled through the inlet into a main 0.5” O.D. stainless steel sampling line maintained at the isokinetic rate of 70 LPM by two venturis mounted on the fuselage in the aft section of the aircraft.  The aerosol instruments subsampled from the main sampling line.

AMS  The high resolution aerosol mass spectrometer (AMS) (Aerodyne) measures submicron particles that are sampled through a critical orifice and focussed through an aerodynamic lens into a region of low vacuum.  The particles impact a heated surface (600 °C), are vapourized and ionized by 70eV impaction.  Ions are then transferred to a time-of-flight mass spectrometer (Tofwerk) where they are accelerated by electric fields and separated by their velocities which are dependent on their mass to charge ratios.  Ions are then detected by charged microchannel plates.  The AMS was operated only in V mode with 10 sec time resolution.   Several ionization efficiency calibrations performed prior to and during the field campaign varied by <10 %.  To determine the AMS collection efficiency, number concentrations measured by an Ultra High Sensitivity Aerosol Spectrometer (UHSAS; Droplet Measurement Technologies Inc.) over a size range of 60 nm to 1 µm were converted to volume concentrations using mid-point bin diameters and assuming spherical shapes.  Total mass was calculated from the UHSAS measurements based on the composition-weighted proportional density determined from the AMS.  A collection efficiency of 0.5 was determined.  Detailed investigations and discussions around the collection efficiency of the AMS can be found in the literature (Middlebrook et al., 2012; Dunlea et al., 2009; Kleinman et al., 2008; Drewnick et al., 2004; Quinn et al., 2006). PM1 is the sum of the mass concentrations of AMS components (OA, NO3, SO4 and NH4).  The PM1 from this study is compared with the PM2.5 EF from the Andreae (2019) literature review of boreal forest wildfire studies.  While these measurements represent PM over different size ranges (< 1µm vs <2.5µm), the difference is not expected to be significant based on typical size distributions of wildfire emissions (Andreae 2019; Reid and Hobbs, 1998; Reid et al., 2005).  This approach has been used previously in literature reviews of EFs for wildfire emissions (Andreae 2019; Akagi et al., 2011).

BC  The SP2 measures the mass of rBC contained in individual aerosols through the laser-induced incandescence of heated rBC-containing aerosols (Stephens et al., 2003; Baumgardner et al., 2004; Schwarz et al., 2006).  The SP2 was calibrated using fullerene soot (Alpha Aesar lot# F12S011) (Moteki and Kondo, 2010; Kondo et al., 2011; Laborde et al., 2012) nebulized from a water suspension and passed through an aerosol particle mass analyzer (Kanomax APM3600) to select particles with masses ranging from 0.2 fg/particle to 48 fg/particle.  Extremely large particles containing more than 520 fg of rBC were excluded from analysis due to saturation of the detector (these accounted for only 2x10-3 % of the total number of rBC containing particles measured by the SP2).

UHSAS  Particle size distributions were measured using an Ultra-High Sensitivity Aerosol Spectrometer.  The UHSAS measures the size of individual aerosols passing through a laser beam via Mie scattering (Cai et al., 2008; Kupc et al., 2018).  These particles are classified into 99 log-normally spaced bins across the measurement range.  Periods where the particle concentration measured by the UHSAS exceeded 3000 particles s-1 were excluded from this analysis due to the potential of coincident particles passing through the laser beam.  The UHSAS particle sizing was verified using NIST traceable polystyrene latex (PSL) nanospheres of multiple sizes across the measurement range.  Total particle mass was calculated from the UHSAS measurements assuming a density of 1.2 g cm-3, based on the composition-weighted proportional density determined from the AMS.

1.1.3 Identification of organic compounds

Three methods were used to provide detailed measurements of gas-phase organic compounds that included the PTRMS, CIMS, and canister samples (AWAS).  The PTRMS and CIMS are able to resolve the molecular formula of isobaric species, but cannot distinguish isomers, while the AWAS system can identify and speciate individual compounds.  For the PTRMS measurements, compound molecular formulae were assigned based on a limited set of possibilities (particularly for the smaller compounds), known or expected compound sensitivities, and comparing with previously published laboratory work by Koss et al. (2018) based on typical NMOG structures observed in biomass burning emissions.  Koss et al. (2018) used a combination of gas-chromatography (GC) pre-separation, NO+ CIMS and time series correlations to identify 156 compounds measured in biomass burning laboratory experiments with a PTRMS.  Additional comparisons were made with PTRMS ion masses reported in Permar et al. (2021) where they used a PTRMS with two GC methods to speciate isomers for some PTRMS ion masses.  For formulas with multiple isomer contributions that were not speciated by the PTRMS, or provided by the AWAS, the fractional contributions in Koss et al. (2018) and Permar et al. (2021) were used to identify the dominant ion and/or contributing compounds.  Although the Koss et al. (2018) work was based on laboratory measurements, Permar et al. (2021) found that isomer contributions did not vary much between 24 fires types across the WE-CAN airborne field campaign in western US, which were mainly dominated by fires of pine, fir and spruce trees.  For example, in Koss et al. (2018), for the mass spectral ion of C3H6O (m/z 58.08), the contribution from acetone was set at 100 % and propanal 0 %, only slightly different from the contribution of 83±6 % for acetone determined by Permar et al., 2021; thus this compound was identified as acetone in this work.  Another example is C4H8O at m/z 70.091 that has potential contributions from methyl vinyl ketone (MVK), methacrolein and crotonaldehyde, both Koss et al. (2018) and Permar et al. (2021) both reported that MVK is the largest contributor at 48 % and 60±9 % respectively; the compound was labelled here as all three.  For C8H10 (m/z 106.168), there are contributions from ethylbenzene, m- and p-xylenes and o-xylene identified as 10%, 68% and 23%, respectively (Koss et al., 2018), with slightly different isomer contributions as per Permar et al. (2021).  In this study, as it was not possible to speciate C8H10 with the AWAS system, it is simply identified as C8 aromatics.  Similarly, C10H16, (m/z 136.238) is identified as total monoterpenes in the present study, with expected contributions from multiple species including alpha and beta pinene, camphene, myrcene, and tricyclene (Permar et al., 2021; Hatch et al., 2017).  C5H8 at m/z 68.119 was identified as isoprene in the present study, recognizing that there may be a fractional contribution to this mass from methyl-3-buten-2-ol (MBO), although Permar et al. (2021), suggests that MBO may not be significant, based on their analysis of western US wildfires.

For the CIMS, the iodide reagent ion chemistry is most sensitive to polar compounds, particularly carboxylic acids and less sensitive to non-polar compounds (Table S1). Compounds were identified using these known/expected sensitivities and available calibration standards.  The AWAS provided speciated measurements of hydrocarbons (≤C10), and no oxygenates.

1.1.4 Overlapping compounds   There were a number of compounds (or molecular formulae) that were measured by both the PTRMS and the AWAS, as well as compounds that overlapped between the PTRMS and CIMS.  Tables S4 and S5 summarize the decisions of overlapping compounds that were retained for derivation of EFs and ERs, as well as for the carbon/nitrogen budget analyses.  For the PTRMS and AWAS, for some molecular formulae, the AWAS provided measurements of individual isomers, while the corresponding PTRMS measurement was expected to reflect the sum of multiple isomers.  Comparisons between these two methods are challenging due to the influence of isomers in the PTRMS signal, and the fact that a number of the PTRMS compounds are determined using calculated sensitivities (i.e. not directly calibrated with a standard) with estimated uncertainties of 50 %.  These factors limit a comparison between the AWAS and PTRMS to isoprene (C5H8) which is shown in Fig. S3a.  The comparison for Screens 1 through 3 shows good agreement with an r2=0.87.  When including only Screen 1 data, there are two data points (in the SP) where the PTRMS is a factor of 2.5 to 3 higher than the AWAS, resulting in a lower r2=0.45.  Although the PTRMS isoprene signal is known to have interferences from cycloalkanes, these compounds are not expected to be emitted from wildfires.  2-methyl-3-buten-2-ol (MBO) produces a fragment at m/z 69.070 that is not separated in the PTRMS, and can also interfere with the isoprene measurement.  We do not have measurements to confirm the impact of MBO on the isoprene signal.  However, Permar et al. (2021) reported that PTRMS derived isoprene measurements during their study were approximately 2x higher than the AWAS isoprene while sampling smoke, but MBO which was measured during that study was considered too low to account for the higher than expected isoprene.  In the present study, it is possible that there were contributions from other unknown isomers to the PTRMS signal in the fresh smoke plumes along Screen 1.  Due to these uncertainties, and the comparatively fewer in-plume AWAS samples, EFs and ERs for isoprene are reported from both the PTRMS and AWAS, and isoprene from the PTRMS was used in the carbon budget (Table S4).

In deriving EFs and ERs, both the PTRMS and AWAS measurements were included to retain as much information as possible.  To avoid double-counting compounds in the carbon and nitrogen budget analyses, only the PTRMS measurement was typically included as it is expected to reflect a sum of multiple isomers, thus, accounting for more carbon. For example, at the molecular formulas of C5H10, the PTRMS measurements are expected to reflect the sum of all the isomers, whereas 7 compounds were speciated from the AWAS i.e. c-2-pentene, cyclopentane, 1-pentene, 2-me-1-butene, 3-methyl-1-butene, t-2 pentene, and 2-me-2-butene.  In this case, EFs and ERs were derived for both the PTRMS and AWAS measurements, but only the PTRMS measurements were included in the budget analyses.

Between the PTRMS and CIMS, there were 18 overlapping molecular formulae.  Although comparisons of exact masses between the PTRMS and CIMS are complicated because of the influence of isomers in the PTRMS signal, four exact masses were identified as the same compound including acetic acid, acrylic acid, formic acid, and isocyanic acid.  Figure S3 shows a comparison for the first three compounds, but excludes isocyanic acid as this compound can hydrolyze in the PTRMS drift tube.  The CIMS provided measurements of pyruvic acid, but the PTRMS signal at the same mass is likely affected by inlet line losses.  Acetic acid and acrylic acid show good agreement with r2 > 0.8 with the PTRMS in-plume measurements ~20 % higher than the CIMS (Fig. S3a, b); this is likely due to additional contributions to the PTRMS signal at these respective exact masses.  The comparison for formic acid (Fig. S3c) is poor (r2 =0.3) likely because the PTRMS measurements are noisy and have a high detection limit of 2 ppbv, whereas the CIMS detection limit is 0.097 ppbv.  The CIMS measurements were also directly calibrated (Table S2), whereas the PTRMS formic acid sensitivity (and other compounds) were calculated, and as such, the CIMS measurements for the overlapping compounds were retained for analysis (Table S5). .  The remaining overlapping formulae between the CIMS and PTRMS were calibrated with different analytes, and thus assumed to be different species.  While there may in fact be some overlap between isomers contributing to these formulae, their overall contribution to the TC budget is small (<4 %).  An attempt was made to quantify as many other peaks that were present in the CIMS mass spectra as possible and apply sensitivity factors.  However, the available sensitivity factors were based on laboratory experiments investigating anthropogenic emissions and highly uncertain for biomass burning measurements.  Nevertheless, application of these sensitivity factors resulted in average mass from the CIMS spectra totalling < 1.5 % of the TC, so although uncertain, exclusion of these masses is not expected to significantly influence the total carbon budget.  It is assumed that all the acids measured by the CIMS are non-aromatic for classifying into chemical structural categories.

1.2 Mass balance method for estimating aircraft-derived emission rates

The Topdown Emission Rate Retrieval Algorithm (TERRA) was designed to estimate emission rates of pollutants measured by aircraft.  The algorithm is based on the Divergence Theorem to achieve mass balance and been extensively applied to a range of pollutants measured by aircraft (e.g. Hayden et al., 2021; Baray et al., 2018; Li et al., 2017; Liggio et al., 2017; Gordon et al., 2015).  An extension of TERRA previously used for estimating pollutant mass transfer rates across virtual aircraft screens (e.g. Hayden et al., 2021; Baray et al., 2018) was used in the present study to estimate emission rates of NOx, CO, PM1 and NH3 using Screen 1 data.  Briefly, pollutant concentrations and horizontal wind speeds were mapped to virtual screens and interpolated using a kriging function, as well as extrapolating the measurements from the lowest aircraft altitude to the surface using applicable extrapolation profiles.  Mass transfer rates were derived by integration of the horizontal fluxes across the plume on the screen in units of t h-1.  The main uncertainty in the mass transfer rate is due to extrapolation to the surface as described previously (e.g. Gordon et al., 2015 and Hayden et al., 2021).

2.0  Flight and fire description

The 18BN-Larry fire (the Lac LaLoche fires) was detected by the Saskatchewan wildfire management agency on June 23, 2018.  Satellite images showed fire hot spots on June 23 and by the evening of June 23 it grew to 1,250 hectares (ha) and to 2,600 ha on June 24th.  On the morning of June 25, 2018, there was a very weak nocturnal inversion and moderately strong south to southeast winds at 33 knots above the inversion.  The range of fire intensities during the previous night, as well as the observed high humidity (80-90 %) and light to moderate winds observed at the surface also indicates that the fire source at the time of aircraft sampling was in a smoldering combustion state.  Flight tracks were flown at Lagrangian distances downwind of the wildfire.  Multiple passes (i.e. transects) perpendicular to the plume direction were made at different altitudes.  Two plumes were identified: the SP was clearly due to the fire hot spots identified by MODIS (green dots encompassed by a polygon), but the source of the NP is less certain.  It is possible that MODIS was unable to detect the fire source because the fire heat signature was below the threshold for satellite instrument detection.  However, surface wind measurements at Lac LaLoche (SI Table S1 in McLagan et al., 2021) show that the wind direction was southerly just prior (approx. 30 min) to the start of the aircraft measurements, and then shifted to southeast (135+/-13°) for the duration of the aircraft flight.  Therefore, it is likely that the NP was from the same fire source as the SP that had been transported in a northerly direction just prior to aircraft sampling, and subsequently moved to a northwesterly direction with the wind change.  The width of the SP and NP was approximately 14-37 km separated by approximately 8-19 km depending on the sampling altitude, and with an aircraft speed of ~90 m s-1, the plumes were traversed in 3-7 min (Fig. 2).

3.0 Combustion state

The combustion efficiency (CE) can be used to characterize and compare the combustion state of the fire, (i.e. the fraction of fuel carbon converted to atmospheric CO2) (Ward and Radke, 1993).  The CE is dependent upon many factors including fire combustion state, fuel chemistry, fuel geometry, growth stage, moisture content and meteorological conditions such as wind speed and temperature.  In a flaming fire, high temperature reactions tend to go to completion (>90 %) as rapid reaction of O2 with the fuel C, H, N and S produces highly oxidized gases including CO2, NOx, and SO2 and BC.  As a fire progresses, incomplete combustion characteristics of smoldering fires becomes more dominant resulting in a larger proportion of the emitted carbon in the form of CO, CH4, NMOC, and OA.  Airborne measurements tend to sample a mixture of combustion states; however, there tends to be a dominant phase of combustion at different fire stages (Andreae and Merlet, 2001).  If only accounting for CO2 and CO, the MCE for the SP is 0.90±0.01 and NP is 0.88±0.01, higher than the CE by 7.1 % and 6.6 % for the SP and NP, respectively.  These differences are driven mostly by the additional contribution from NMOGs indicating the importance of their inclusion in assessing fire combustion state.

Since the flaming phase was likely more than 14 hrs prior to aircraft sampling, it is possible that the emissions from this fire may also reflect a residual smoldering combustion (RSC) component.  RSC produces emissions from combustion of forest floor and woody debris that are not associated with flaming, can be sustained for long periods of time after the passage of a flame front, and are not strongly lofted through fire-induced convection (Bertschi et al., 2003).  It is noted though that observations of increased levels of flaming compounds in the plumes including CO2 and BC (Fig. S4) suggest that to some extent, flaming processes also contributed to the release of these compounds.  It is likely that different parts of the fire had varying mixtures of smoldering, flaming and residual combustion processes, but the evidence strongly suggests that the Lac LaLoche fire was predominantly in a smoldering combustion state during the aircraft measurement time period.  Smoldering fires can create persistent and poorly ventilated smoke that can be a significant driver in remote community evacuations (McGee, 2020).  In addition, boreal fires in this region tend to exhibit a large component of smoldering combustion which can consume large amounts of above and below ground biomass (Akagi et al., 2011).

Table S1. Measurements with associated instrument, principle of operation, sampling time resolution, uncertainty, detection limit, and applicable method references.

| Measurement | Instrument | Principle of measurement | Time Resolution (sec) | Uncertainty | Detection Limit | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| NO | Thermo 42i | Chemiluminescence with O3 | 1 | ±5 % | 3σ at 1 sec 0.17 ppbv | Clyne at al., 1964 Ridley et al., 1990 |
| NO2 | Thermo 42i | Photolysis + chemiluminescence with O3 | 1 | ±7 % | 3σ at 1 sec 0.43 ppbv | Penkett et al., 2011 |
| NOy | Thermo 42i | Heated (350°C) and molybdenum catalyzed conversion + chemiluminescence with O3 | 1 | ±5 % | 3σ at 1 sec 0.18 ppbv | Fehsenfeld et al., 1987; Williams et al., 1998 |
| SO2 | Thermo 43i | UV pulsed fluorescence | 1 | ±5 % | 3σ at 1 sec 1.25 ppbv | Stecher et al., 1997 |
| O3 | Thermo 42i | Chemiluminescence | 1 | ±5 % | 3σ at 1 sec 0.52 ppbv | N/A |
| NH3 | LGR model 911-0039 | Absorption | 1 | ±5 % | 2.1 ppbv at 1 sec 0.3 ppbv at 60 sec | Leifer et al., 2017 |
| Hg (GEM) | Tekran 237X | Fluorescence | 120 | ±0.054 ng m-3 | <0.1 ng m-3 | Cole et al., 2014; McLagan et al., 2021 |
| CO, CO2, CH4 | Picarro G-2401-m | Cavity ring down spectrometry | 2 | CH4 ~±3 ppbv | N/A | Baray et al., 2018 |
| Total Carbon (TC) | Picarro G-2401-m | Heated (650°C) platinum catalyzed conversion to CO2 | 2 | 3σ at 10 sec ±60 ppbv | N/A | Stockwell et al., 2018; Veres et al., 2010. |
| Total non-methane organic gases (NMOGT) | Picarro G-2401-m x 2 | Difference method, heated (650°C) platinum catalyzed conversion to CO2 | 10 | 3σ at 10 sec ±85 ppbv | 85 ppbv 3σ at 10 sec | Stockwell et al., 2018; Veres et al., 2010 |
| VOCs | CIMS | Chemical ionization/mass spectrometry | 1 | ±10-50% compound dependent | See Table S2 | Liggio et al., 2017; Lee et al., 2014 |
| VOCs (≤C10) | AWAS | Grab samples/GC with MS and FID analysis | 20-30 | <C6 ±25% ≥C6 ±40% | 0.7 to 12.9 pptv for compounds from C2 to C8; 1 to 242 pptv for C9 and C10 compounds | N/A |
| VOCs | PTRMS | Proton transfer/ionization/mass spectrometry | 1 | VOCs with available standards: ±15% except CH2O at±20%; Calculated VOCs: ±50% | Range of 0.005 to 1 ppbv, CH2O 4.6 ppbv at 1 sec | Li et al., 2017; Sekimoto et al., 2017 |
| VOCs-SVOCs (C10-C25) | Custom packed adsorbent tubes | Offline analysis with TD-GC-APCI-Q-ToF | Variable (245-3140 sec per tube) |  | 1 ppt | Sheu et al., 2018; Khare et al., 2019; Ditto et al., 2021 |
| Particle chemical composition | Aerosol mass spectrometer | Volatilization, ionization and mass spectrometry | 10 | OA: ±35% SO4: ±25% NO3: ±20% NH4: ±25% | OA:0.24 µg m-3 SO4: 0.05 µg m-3 NO3: 0.035 µg m-3 NH4: 0.15 µg m-3 at 10 sec | DeCarlo et al., 2008; Jimenez et al., 2003; Allan et al., 2003 |
| Black carbon | SP2 | Incandescence | 1 | 15 % | 0.01 µg m-3 at 1 sec | Stephens et al., 2003; Baumgardner et al., 2004; Schwarz et al., 2006 |
| Particle size distributions (60 -1000 nm) | Ultra-High Sensitivity Aerosol Spectrometer (UHSAS) | Particle light scattering | 1 | 10 % |  | Cai et al., 2008; Kupc et al., 2018 |

Table S2. Standards used to calibrate the CIMS, as well as the compound uncertainty, detection limit and applicable method reference.  The iodide chemistry is most sensitive to polar compounds and less sensitive to non-polar compounds.  The sensitivity tends to increase for keto-, hydroxy- and acid groups, in order. Most of the keto- groups are attached to a carboxylic acid. For the larger acids (>C4) where there can be several isomers, they are generally identified as saturated C4 carboxylic acids and unsaturated C5 acids.

| Molecular weight | Molecular formula | Compound Name | Calibration standard | Calibration Source | Uncertainty (%) | Detection Limit (pptv) | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 27.026 | HCN | hydrogen cyanide | Hydrogen Cyanide | High Pressure Cylinder (Air Liquide) | 20 | 17 | Stockwell et al 2018 |
| 32.06 | SO2 | sulphur dioxide | Sulfur Dioxide | High Pressure Cylinder (Air Liquide) | N/A | N/A | Lee et al 2018 |
| 43.025 | HNCO | isocyanic acid | Isocyanic Acid | Thermal Decomposition/Diffusion | 30 | 10 | Roberts et al 2010 |
| 46.025 | CH2O2 | formic acid | Formic Acid | Liquid Standard supplied by Liquid Calibration Unit (LCU) | 20 | 156 | Mungall et al 2017 |
| 47.013 | HONO | nitrous acid | Nitrous Acid | Acid Displacement (output quantified by ion chromatography) | 30 | 12 | Roberts et al 2010 |
| 57.052 | C2H3NO | hydroxy acetonitrile | Glycolic Acid Nitrile | Liquid Standard supplied by Liquid Calibration Unit (LCU) | 20 | 0.12 | Mungall et al 2017 |
| 60.052 | C2H4O2 | acetic acid | Acetic Acid | Liquid Standard supplied by Liquid Calibration Unit (LCU) | 20 | 576 | Mungall et al 2017 |
| 63.012 | HNO3 | nitric acid | Nitric Acid | Permeation Tube (output quantified by ion chromatography) | N/A | 58 | Neuman et al 1999 |
| 72.063 | C3H4O2 | acrylic acid | Acrylic Acid | Liquid Standard supplied by Liquid Calibration Unit (LCU) | 20 | 17 | Mungall et al 2017 |
| 74.079 | C3H6O2 | propionic acid | Propionic Acid | Liquid Standard supplied by Liquid Calibration Unit (LCU) | 20 | 55 | Mungall et al 2017 |
| 79.011 | HNO4 | pernitric acid | Pernitric Acid | Reaction of HO2 with NO2 (quantified by thermal decomposition Cavity Ringdown Spectroscopy of NO2) | 50 | 2 | Veres et al 2015 |
| 84.074 | C4H4O2 | C4H4O2 | 2(5H)-Furanone | Liquid Standard supplied by Liquid Calibration Unit (LCU) | 20 | 51 | Mungall et al 2017 |
| 88.062 | C3H4O3 | pyruvic acid | Pyruvic Acid | Liquid Standard supplied by Liquid Calibration Unit (LCU) | 20 | 253 | Mungall et al 2017 |
| 100.117 | C5H8O2 | unsaturated C5 carboxylic acids | 4-Pentenoic Acid | Liquid Standard supplied by Liquid Calibration Unit (LCU) | 20 | 19 | Mungall et al 2017 |
| 102.089 | C4H6O3 | C4 oxo-carboxylic acids | 2-Ketobutyric Acid | Liquid Standard supplied by Liquid Calibration Unit (LCU) | 20 | 354 | Mungall et al 2017 |
| 114.144 | C6H10O2 | sum of cyclic saturated and n-unsaturated C5 carboxylic acids | Cyclopentanecarboxylic Acid | Liquid Standard supplied by Liquid Calibration Unit (LCU) | 20 | 28 | Mungall et al 2017 |
| 116.116 | C5H8O3 | C5 oxo-carboxylic acids | Levulinic Acid | Liquid Standard supplied by Liquid Calibration Unit (LCU) | 20 | 11 | Mungall et al 2017 |
| 126.155 | C7H10O2 | unsaturated C6 cyclic carboxylic acid | 3-Cyclohexene-1-carboxylic Acid | Liquid Standard supplied by Liquid Calibration Unit (LCU) | 20 | 6 |  |
| 132.159 | C6H12O3 | C6 hydroxy-carboxylic acids | 2-Hydroxyisocaproic Acid | Liquid Standard supplied by Liquid Calibration Unit (LCU) | 20 | 1.6 |  |

Table S3. Standards used to calibrate the PTRMS.  aC8 aromatics - expected contributions from ethyl benzene, m- and p-xylenes and o-xylene. bMonoterpenes - expected contributions from camphene, α-pinene, β-pinene, myrcene, and tricyclene.

| Molecular weight | Molecular formula | Compound Name | Calibration standard |
| --- | --- | --- | --- |
| 30.026 | CH2O | formaldehyde | formaldehyde |
| 32.042 | CH4O | methanol | methanol |
| 34.076 | H2S | hydrogen sulfide | hydrogen sulfide |
| 41.053 | C2H3N | acetonitrile | acetonitrile |
| 44.053 | C2H4O | acetaldehyde | acetaldehyde |
| 56.064 | C3H4O | acrolein | acrolein |
| 58.08 | C3H6O | acetone | acetone |
| 62.13 | C2H6S | dimethyl sulfide | dimethyl sulfide |
| 68.119 | C5H8 | isoprene | isoprene |
| 70.091 | C4H6O | MVK, methacrolein, crotonaldehyde | crotonaldehyde |
| 72.107 | C4H8O | MEK, 2-methyl acetate, ethyl formate | methylethyl ketone |
| 76.157 | C3H8S | 2-propanethiol, ethyl methyl sulfide | ethylmethyl sulfide |
| 78.114 | C6H6 | benzene | benzene |
| 84.136 | C4H4S | thiophene | thiophene |
| 90.184 | C4H10S | diethyl sulfide, butanethiol | diethyl sulfide |
| 92.141 | C7H8 | toluene | toluene |
| 98.163 | C5H6S | methyl thiophene | 2-methylthiophene |
| 106.168 | C8H10 | C8 aromaticsa | o-xylene |
| 112.19 | C6H8S | dimethylthiophene | 2,3-dimethylthiophene |
| 136.238 | C10H16 | monoterpenesb | camphene |

Table S4. Overlapping compounds measured between the PTRMS and AWAS.

| Molecular Weight | Formula | Compound Name | Instrument | Decision for carbon budget | Decision for EFs |
| --- | --- | --- | --- | --- | --- |
| 42.081 | C3H6 | propene | AWAS and PTRMS | AWAS | AWAS |
| 54.092 | C4H6 | butadiene/fragments | PTRMS | PTRMS | PTRMS and AWAS |
|  |  | 1,3-butadiene | AWAS |  |  |
| 56.108 | C4H8 | butenes | PTRMS | AWAS | AWAS |
|  |  | cis-2-butene | AWAS |  |  |
|  |  | isobutene | AWAS |  |  |
|  |  | trans-2-butene | AWAS |  |  |
|  |  | 1-butene | AWAS |  |  |
| 58.124 | C4H10 | butanes | PTRMS | AWAS | AWAS |
|  |  | n-butane | AWAS |  |  |
| 68.119 | C5H8 | isoprene | PTRMS and AWAS | PTRMS | PTRMS and AWAS |
| 70.135 | C5H10 | pentene/fragments | PTRMS | PTRMS | PTRMS and AWAS |
|  |  | cis-2-pentene | AWAS |  |  |
|  |  | cyclopentane | AWAS |  |  |
|  |  | 1-pentene | AWAS |  |  |
|  |  | 2-methyl-1-butene | AWAS |  |  |
|  |  | 2-methyl-2-butene | AWAS |  |  |
|  |  | 3-methyl-1-butene | AWAS |  |  |
|  |  | t-2 pentene | AWAS |  |  |
| 82.146 | C6H10 | cyclohexene | PTRMS and AWAS | PTRMS | PTRMS |
| 84.162 | C6H12 | hexene | PTRMS | PTRMS | PTRMS and AWAS |
|  |  | cyclohexane | AWAS |  |  |
|  |  | cis-2-hexene | AWAS |  |  |
| 86.178 | C6H14 | hexanes | PTRMS | AWAS | AWAS |
|  |  | n-hexane | AWAS |  |  |
|  |  | 2,3-dimethyl butane | AWAS |  |  |
|  |  | 2,3-dimethylpentane | AWAS |  |  |

Table S5. Overlapping compounds measured between the PTRMS and CIMS.

| Molecular weight | Formula | CIMS compound name | PTRMS compound name | Decision |
| --- | --- | --- | --- | --- |
| 43.025 | HNCO | Isocyanic acid | Isocyanic acid | CIMS and PTRMS for reporting EF and ER; CIMS for carbon budget |
| 46.025 | CH2O2 | Formic acid | Formic acid | CIMS |
| 57.052 | C2H3NO | Hydroxy acetonitrile | Methyl isocyanate | CIMS and PTRMS |
| 60.052 | C2H4O2 | Acetic acid | Acetic acid | CIMS |
| 72.063 | C3H4O2 | Acrylic acid | Methyl glyoxal/acrylic acid | CIMS |
| 74.079 | C3H6O2 | Propionic acid | Hydroxy acetone/ethyl formate | CIMS and PTRMS for reporting EF and ER; CIMS for carbon budget |
| 84.074 | C4H4O2 | Unidentified | Furanone | CIMS and PTRMS for reporting EF and ER; CIMS for carbon budget |
| 85.062 | C3H3NO2 | Cyanoacetic acid | Methyl cyanoformate | No significant emissions |
| 86.09 | C4H6O2 | Methacrylic acid | Butanedione/isomers | PTRMS |
| 88.106 | C4H8O2 | C4 saturated carboxylic acids | Methyl propanoate | PTRMS |
| 100.117 | C5H8O2 | Unsaturated C5 carboxylic acids | Methyl methacrylate/isomers | CIMS and PTRMS for reporting EF and ER; CIMS for carbon budget |
| 102.089 | C4H6O3 | C4 oxo-carboxylic acids | Acetic anhydride | CIMS and PTRMS for reporting EF and ER; CIMS for carbon budget |
| 102.133 | C5H10O2 | C5 saturated carboxylic acids | Valeric acid | PTRMS |
| 114.144 | C6H10O2 | Sum of cyclic saturated and n-saturated C5 carboxylic acids | Caprolactone/c6 esters/c6diketone isomers | CIMS and PTRMS for reporting EF and ER; CIMS for carbon budget |
| 116.16 | C6H12O2 | C6 carboxylic acids | Butyl acetate/c6 esters | PTRMS |
| 118.132 | C7H10O2 | Unsaturated C6 cyclic carboxylic acids | Cyclohexene carboxylic acid | No significant emissions |
| 128.171 | C7H12O2 | C6 unsaturated carboxylic acids | Cyclohexanoic acid | PTRMS |
| 130.187 | C7H14O2 | C7 saturated carboxylic acids | Amyl acetate | PTRMS |

Table S6. Compounds with no significant observed emissions

| Molecular Weight | Formula | Compound Name | Instrument |
| --- | --- | --- | --- |
| 34.08 | H2S | hydrogen sulfide | PTRMS |
| 70.05 | C2H3O2 | Propiolic acid | PTRMS |
| 72.17 | C5H12 | 2,2-dimethylpropane | AWAS |
| 82.06 | C4H2O2 | cyclobutenedione | PTRMS |
| 85.06 | C3H3NO2 | cyanoacetic acid | CIMS |
| 86.2 | C6H14 | 2,2-dimethylbutane | AWAS |
| 91.07 | C2H5NO3 | C2 nitro alcohol | CIMS |
| 100.07 | C4H4O3 | dihydro furandione | PTRMS |
| 102.195 | C5H10S | cyclopentanethiol | PTRMS |
| 104.105 | C4H8O3 | C4 hydroxy-carboxylic acids | CIMS |
| 112.24 | C8H16 | cis-1,2-dimethylcyclohexane | AWAS |
| 112.56 | C6H5Cl | chlorobenzene | PTRMS |
| 118.13 | C5H10O3 | C5 hydroxy-carboxylic acids | CIMS |
| 128.29 | C9H20 | 2,5-dimethylheptane | AWAS |
| 134.24 | C10H14 | 1-methyl-2-n-propylbenzene | AWAS |
| 140.25 | C8H12S | butylthiophene | PTRMS |
| 142.32 | C10H22 | 2,2-dimethyloctane | AWAS |
| 147.00 | C6H4Cl2 | dichlorobenzene | PTRMS |
| 154.12 | C7H6O4 | dihydroxybenzoic acid | PTRMS |
| n/a | Cl- | p-chloride | AMS |

Table S7.  Emission ratios (relative to CO, as µg m-3 of I/SVOC per µg m-3 CO) for complex mixtures of gas-phase CH, CHO1, and CHS1 compounds grouped by carbon number for all targeted molecular formulas, derived from the integrated cartridge samples, using the sample taken across the lowest transects of Screen 1. The ERs are reported as a range with the lower limit reflecting the subtraction of a slightly contaminated background, and the upper limit having no background subtracted.

| Carbon Number | CH (µg m-3)/ CO (µg m-3) | CHO1 (µg m-3)/ CO (µg m-3) | CHS1 (µg m-3)/ CO (µg m-3) |
| --- | --- | --- | --- |
| 10 | 1.1E-02 - 1.1E-02 | 3.2E-03 - 3.2E-03 | 7.2E-06 - 7.9E-06 |
| 11 | 0.0E+00 - 0.0E+00 | 6.6E-04 - 6.6E-04 | 1.6E-05 - 1.6E-05 |
| 12 | 5.4E-05 - 5.4E-05 | 4.6E-04 - 7.4E-04 | 0.0E+00 - 2.4E-06 |
| 13 | 3.3E-06 - 2.4E-04 | 4.4E-05 - 5.4E-04 | 9.2E-06 - 1.4E-05 |
| 14 | 5.3E-05 - 1.6E-04 | 3.7E-04 - 1.1E-03 | 4.4E-06 - 5.8E-06 |
| 15 | 6.0E-05- 3.7E-04 | 3.3E-04 - 1.0E-03 | 2.2E-05 - 2.4E-05 |
| 16 | 3.4E-04 - 1.1E-03 | 3.6E-04 – 7.0E-04 | 5.4E-05 - 6.8E-05 |
| 17 | 7.6E-04 - 1.2E-03 | 1.2E-03 - 1.9E-03 | 3.3E-04 - 3.4E-04 |
| 18 | 1.0E-03 - 1.7E-03 | 2.1E-04 - 1.3E-03 | 1.2E-04 - 1.2E-04 |
| 19 | 1.6E-03 - 1.9E-03 | 5.7E-04 -1.2E-03 | 3.9E-04 - 4.3E-04 |
| 20 | 1.9E-03 - 2.8E-03 | 3.1E-04 - 9.1E-04 | 1.1E-04 - 1.4E-04 |
| 21 | 2.5E-03 - 2.5E-03 | 2.6E-04 - 5.2E-04 | 2.3E-04 - 2.9E-04 |
| 22 | 3.8E-03 - 4.0E-03 | 1.7E-04 - 3.3E-04 | 1.9E-05 - 8.9E-05 |
| 23 | 2.1E-03 - 2.2E-03 | 6.7E-05 - 1.5E-04 | 2.9E-05 - 2.9E-05 |
| 24 | 7.6E-04 - 1.0E-03 | 3.3E-05 - 1.2E-04 | 4.4E-05 - 6.0E-05 |
| 25 | 4.0E-04 - 4.3E-04 | 9.0E-06 - 5.5E-05 | 1.3E-05 - 6.1E-05 |

Table S8. Compounds shown in Fig. 9 where the identifications/naming are not exact matches with the current study. 1Individually identified compounds were summed for comparison to the present study; 2Each compound was compared with the value from the present study.

| Molecular Weight | Compound | Instrument | Compound Name | Andreae Names | Koss Names | Permar Names | Urbanski Names |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 54.092 | C4H6 | PTRMS | butadiene/fragments | butadiene | 1,3-butadiene + 1,2-butadiene | 1,3-butadiene, 1,2-butadiene | n/a |
| 54.092 | C4H6 | AWAS | 1,3-butadiene | butadiene | 1,3-butadiene + 1,2-butadiene | 1,3-butadiene, 1,2-butadiene | n/a |
| 57.052 | C2H3NO | CIMS | hydroxy acetonitrile | n/a | methyl isocyanate + hydroxyacetonitrile | methyl isocyanate, hydroxyacetonitrile | n/a |
| 58.124 | C4H10 | AWAS | isobutane | n/a | n/a | n-Butane | n/a |
| 60.052 | C2H4O2 | CIMS | acetic acid | Acetic acid | acetic acid + glycolaldehyde | acetic acid, glycolaldehyde (=hydroxylacetaldehyde) | n/a |
| 60.096 | C3H8O | PTRMS | propanol | n/a | n/a | Isopropanol | n/a |
| 66.103 | C5H6 | PTRMS | cyclopentandiene | n/a | n/a | 1,3-cyclopentadiene | 1,3-CyclopentadienePIT |
| 70.091 | C4H6O | PTRMS | MVK, methacrolein, crotonaldehyde | Methacrolein | MVK + methacrolein + crotonaldehyde | Methyl vinyl ketone, Methacrolein, 2-Butenal (=crotonaldehyde) | 1Crotonaldehyde + Methacrolein + Methyl Vinyl Ketone MVK |
| 70.135 | C5H10 | PTRMS | pentene/methyl butene/fragments | 1-Pentene + 2-pentene | pentene+methyl butene | pentenes, methylbutenes | n/a |
| 70.135 | C5H10 | AWAS | c-2-pentene | 2 pentene cis&tran | pentene+methyl butene | pentenes, methylbutenes | n/a |
| 70.135 | C5H10 | AWAS | cyclopentane |  |  | cyclopentane | n/a |
| 70.135 | C5H10 | AWAS | 1-pentene | 1-Pentene | pentene+methyl butene | pentenes, methylbutenes | n/a |
| 70.135 | C5H10 | AWAS | methyl-1-butene | 1-Pentene | pentene+methyl butene | pentenes, methylbutenes | n/a |
| 70.135 | C5H10 | AWAS | methyl-2-butene | 1-Pentene | pentene+methyl butene | pentenes, methylbutenes | n/a |
| 72.063 | C3H4O2 | CIMS | acrylic acid | n/a | n/a | pyruvaldehyde (=methyl glyoxal), acrylic acid | n/a |
| 72.107 | C4H8O | PTRMS | methy ethyl ketone + butanal + 2-methylpropanal | 2-butanone (methyl ethyl ketone) | methyl ethyl ketone + butanal + 2-methylpropanal | methyl ethyl ketone, 2-methylpropanal, butanal | 1Methyl Ethyl Ketone MEK + n-Butanal + 2-Methylpropanal |
| 72.151 | C5H12 | AWAS | methylbutane | n/a | n/a | n-pentane | n/a |
| 74.079 | C3H6O2 | PTRMS | hydroxy acetone/ethyl formate | n/a | methyl acetate + ethyl formate + hydroxyacetone | Hydroxyacetone, Methyl acetate, Ethyl formate | Ethyl Formate |
| 81.118 | C5H7N | PTRMS | pentene nitriles/methyl pyrrole | n/a | n/a | n/a | 1-Methylpyrrole |
| 82.102 | C5H6O | PTRMS | methyl furan | n/a | 2-methylfuran + 3-methylfuran + general HCO | 2-Methylfuran, 3-Methylfuran | 22-Methylfuran, 3-Methylfuran |
| 84.118 | C5H8O | PTRMS | cyclopentanone/ isomers | n/a | 3-methyl-3-butene-2-one + cyclopentanone + HCO1 isomers | 3-Methyl-3-buten-2-one, Cyclopentanone | Cyclopentanone |
| 84.162 | C6H12 | PTRMS | hexene/fragments | 1-hexene | n/a | n/a | 21-Hexene, cis-2-Hexene |
| 86.09 | C4H6O2 | PTRMS | butanedione/isomers | 2,3-butanedione | 2,3-butanedione + methyl acrylate + other HCO2 | 2,3-butanedione, methyl acrylate | 2,3-Butadione |
| 86.134 | C5H10O | PTRMS | pentanone | n/a | n/a | n/a | 22-Pentanone, 3-Pentanone |
| 86.178 | C6H14 | AWAS | 2+3-methylpentane | n/a | n/a | 3-methylpentane | 3-Methylpentane |
| 96.085 | C5H4O2 | PTRMS | furfural | furfural (2-furaldehyde) | 2-furfural + 3-furfural + other HCO2 | 2-furfural (=furaldehyde), 3-furfural | 2-Furaldehyde |
| 98.189 | C7H14 | PTRMS | heptene | n/a | n/a | n/a | 1-Heptene |
| 100.117 | C5H8O2 | PTRMS | methyl methacrylate/ isomers | n/a | Methyl methacrylate + other HCO2 | Methyl methacrylate | Methyl Methacrylate |
| 100.161 | C6H12O | PTRMS | hexanal/hexanones | n/a | hexanal + hexanones | Hexanones, Hexanal | 1n-Hexanal + Hexanones |
| 103.124 | C7H5N | PTRMS | benzonitrile | n/a | Benzonitrile | Benzonitrile | Benzenenitrile |
| 106.168 | C8H10 | PTRMS | C8 aromatics | n/a | Ethyl benzene + m-xylene + p-xylene + o-xylene | C8 Aromatics | 1Ethylbenzene + m,p-Xylenes + o-Xylene |
| 112.216 | C8H16 | PTRMS | octene | n/a | n/a | n/a | 1-Octene |
| 118.135 | C8H6O | PTRMS | benzofuran | n/a | Benzofuran | Benzofuran | BenzofuranPIT |
| 118.179 | C9H10 | PTRMS | methylstyrenes/ propenyl benzenes | n/a | Indane + methyl styrenes + propenyl benzenes | Methylstyrenes, Indane, Propenylbenzenes | 11-Propenylbenzene, 2-Methylstyrene, 2-Propenylbenzene, 3-Methylstyrene, 4-Methylstyrene, alpha-Methylstyrene |
| 120.195 | C9H | PTRMS | C9 aromatics | 1,2,3- trimethylbenzen, 1,2,4- trimethylbenzen, 1,3,5-trimethylbenzene (Simpson et al., 2011) | C9 aromatics | C9 aromatics | 11,2,3-Trimethylbenzene, 1,2,4-Trimethylbenzene, 1,3,5-Trimethylbenzene, 1-Ethyl-2-Methylbenzene, 1-Ethyl-3-,4-Methylbenzene, Isopropylbenzene, n-Propylbenzene |
| 132.162 | C9H8O | PTRMS | methyl benzo furans | n/a | Methyl benzofuran | Methylbenzofurans | 1Methylbenzofuran isomer 1, Methylbenzofuran isomer 2, Methylbenzofuran isomer 3 |
| 132.206 | C10H12 | PTRMS | ethyl styrene/ methyl propenyl benzene | n/a | Methyl propenyl benzene + ethyl styrene | Ethyl styrenes, Methylpropenylbenzenes, Butenylbenzenes | 11-Methyl-1-Propenylbenzene, Ethylstyrene |
| 134.222 | C10H14 | PTRMS | C10 Aromatics | n/a | C10 Aromatics | C10 Aromatics | 11,4-Diethylbenzene, 1-Butenylbenzene, Ethyl Xylene isomer 1, Ethyl Xylene isomer 2, Isobutylbenzene, Methyl-n-Propylbenzene isomer 1, Methyl-n-Propylbenzene isomer 2, n-Butylbenzene, p-Cymene |
| 136.238 | C10H16 | PTRMS | monoterpenes | sum of alpha + beta-pinene (Simpson et al., 2011) | monoterpenes | monoterpenes | n/a |
| 148.249 | C11H16 | PTRMS | C11 aromatics/ pentamethylbenzene | n/a | n/a | n/a | C11 Aromatics |

Table S9. Emission speciation profile for SAPRC11 chemical mechanism derived from normalized EFs from the present study and compared with wildfire smoldering emission speciation profile from the EPA SPECIATEv4.5 #95428 dataset.  Note that SESQ (sesquiterpene), WSOC (water soluble organic carbon) and IVOC are non-standard SAPRC11 mechanism species.  Please refer to Carter and Heo (2013) for mechanism species definition.

| SAPRC11 Lumped Species Name | Molecular Weight (g/mol) | Normalized Mass Fraction (Hayden et al.) | Normalized Mass Fraction (SPECIATEv4.5 #95428) |
| --- | --- | --- | --- |
| CCOOH | 60.05 | 0.038 | 0.031 |
| ACET | 58.08 | 0.024 | 0.0072 |
| ACYL | 26.04 | 0.0080 | 0.00059 |
| ALK1 | 30.07 | 0.039 | 0.011 |
| ALK2 | 36.73 | 0.013 | 0.0043 |
| ALK3 | 58.61 | 0.0063 | 0.0077 |
| ALK4 | 77.6 | 0.0055 | 0.030 |
| ALK5 | 118.89 | 0.0053 | 0.28 |
| ARO1 | 95.16 | 0.031 | 0.034 |
| ARO2 | 118.72 | 0.042 | 0.067 |
| BACL | 86.09 | 0.021 | 0.0050 |
| BALD | 106.13 | 0.0020 | 0.0034 |
| BENZ | 78.11 | 0.013 | 0.0035 |
| CATL | 110.1 | 0.0032 | 0.014 |
| CCHO | 44.05 | 0.032 | 0.023 |
| CH4 | 16.043 | 0.24 | 0.044 |
| CRES | 108.14 | 0.0026 | 0.0027 |
| ETHE | 28.05 | 0.044 | 0.0065 |
| HCOOH | 46.03 | 0.0050 | 0.0038 |
| GLY |  |  | 0.000046 |
| HCHO | 30.03 | 0.030 | 0.0084 |
| IPRD | 100.12 | 0.00020 | 0.0037 |
| ISOP | 68.12 | 0.021 | 0.00041 |
| MACR | 70.09 | 0.010 | 0.0044 |
| MEK | 72.11 | 0.0066 | 0.0028 |
| MEOH | 32.04 | 0.057 | 0.016 |
| MGLY |  |  | 0.000037 |
| MVK | 70.09 | 0.0057 | 0.019 |
| NROG | 1 | 0.11 | 0.13 |
| NVOL | 1 | 1.18E-05 |  |
| OLE1 | 72.34 | 0.079 | 0.049 |
| OLE2 | 75.78 | 0.019 | 0.025 |
| PACD | 74.08 | 0.055 | 0.00047 |
| PHEN | 94.11 | 0.0055 | 0.0054 |
| PROD2 | 116.16 | 0.0044 | 0.027 |
| RCHO | 58.08 | 0.038 | 0.0035 |
| TERP | 136.24 | 0.024 | 0.032 |
| XYNL | 122.16 | 0.0080 |  |
| SESQ | 204.35 | 0.039 |  |
| WSOC | 227 | 0.013 |  |
| IVOC | 227.3 | 0.0063 |  |

Figure S1. Map showing the home location of the airborne study at Fort McMurray, Alberta and the location of the wildfire in Saskatchewan.  The green shaded region shows the extent of the boreal forest coverage across Canada and Alaska.

Figure S2. Total carbon (TC), Σ(CO2+CO+CH4) (ppmv C), and NMOGT (ppmv C or ppbv C) averaged to 10 sec for a) a portion of the in-flight calibration time period, and b) a selected plume along Screen 1.  NMOGT, the difference between the TC and Σ(CO2+CO+CH4), has an uncertainty of ±85 ppbv C.

Figure S3. Intercomparisons of overlapping compounds between the PTRMS and AWAS, and between the PTRMS and CIMS for a) isoprene, b) acetic acid, c) acrylic acid, and d) formic acid.

Figure S4.  Time series of CO2, BC and NOx mixing ratios, and C2H4O2+ (levoglucosan fragment derived from the AMS) concentrations for Screen 1.  The in-plume portions are indicated by the vertical grey bars.  The aircraft flew back and forth across the plumes at increasing altitudes to complete five transects; a transect represents one pass across the SP and NP at the same altitude.

Figure S5.  Using BC/CO (Selimovic et al., 2019) as an indicator of plume mixing downwind of the Lac LaLoche fire.  The squares show the average and the vertical lines the standard deviation for the transects within the mixed layer for each screen.

Figure S6.  Percent contribution from individually measured particle-phase species for the NP and SP including p-organics (OA), black carbon (BC), ammonium (NH4), sulphate (SO4) and nitrate (NO3), based on mass concentrations.

Figure S7.  Background-subtracted mixing ratios of individually measured NMOGs from the PTRMS, CIMS and AWAS are shown for thirteen chemical classes for the a) NP and b) SP.  In some cases, compounds are double- (or triple-) counted if they can be identified in more than one category.  For example, phenol is an alcohol + an aromatic; guaiacol is an alcohol + an ether + an aromatic.  In the pie chart, the other category includes amides, amines, ethers, thiols and sulfides.  The unidentified category contains molecular formulas detected but the compound(s) could not be identified.

Figure S8.  Background-subtracted average mixing ratios of individually measured NMOGs from the PTRMS, CIMS and AWAS are shown for molecular formulae classes.  The other category contains molecular formulas detected but the compound(s) could not be identified.

Figure S9.  Background-subtracted average mixing ratios of individually measured NMOGs from the PTRMS, CIMS and AWAS by structural group for the a) NP and b) SP.  The other category is the sum of terpenes, phenols, pyridines, pyrroles and thiophenes.

Figure S10.  Percent contributions of carbon-containing compounds to the TC based on EFs (in terms of carbon fraction) for the SP and NP.  The two pie charts on the right, representing the low and high I/SVOC EF estimates, show the percent breakdown of the measured NMOGs and the remaining unidentified portion.  The EF values (g C kg-1) are identified in the boxes.  Note, the I/SVOC measurements represent the integrated average encompassing both plumes.

Figure S11.  Emission factors (EF) (g kg-1) for the SP and NP determined from measurements of a) particle species; and b) inorganic gas-phase species.

Figure S12.  Emission ratios (ER) for the SP and NP determined from measurements of a) particle species (µg m-3 ppm-1); and b) inorganic gas-phase species (ppb ppm-1).

Figure S13.  Emission factors (EF) (g kg-1) for the a) NP and b) SP for the top 25 measured gas-phase organic species.  C4 acids = C4 oxo-carboxylic acids, propadiene = fragments/propadiene, hydroxy acetone = hydroxy acetone/ ethyl formate; MEK = MEK, butanal and 2-methylpropanal .

Figure S14.  Comparison of the normalized organic gas speciation profile derived from this study with that from the EPA’s SPECIATE4.5 (#95428) profile.   EFs in the present study were mapped to the SAPRC-11 model mechanism species and normalized to total organic gas (which does not include the unidentified mass fraction), to create a total organic gas mass speciation profile.  The total organic mass speciation profile is plotted against the similarly treated mass speciation profile from the EPA SPECIATEv4.5 #95428 for wildfire smoldering emissions.  Note that for comparison purposes the non-standard SAPRC-11 species in the present study are lumped, such that SESQ is summed with TERP, and IVOC, WSOC and NVOL are summed with NROG.

References

Akagi, S. K., Yokelson, R. J., Wiedinmyer, C., Alvarado, M. J., Reid, J. S., Karl, T., Crounse, J. D., and Wennberg, P. O.: Emission factors for open and domestic biomass burning for use in atmospheric models, Atmos. Chem. Phys., 11, 4039-4072, https://doi.org/10.5194/acp-11-4039-2011, 2011.

Allan, J. D., Jimenez, J. L., Williams, P. I., Alfarra, M. R., Bower, K. N., Jayne, J. T., Coe, H., and Worsnop, D. R.: Quantitative sampling using an Aerodyne Aerosol Mass Spectrometer 1. Techniques of data interpretation and error analysis, J. Geophys. Res.-Atmos, 108, 4090, https://doi.org/10.1029/2002JD002358, 2003.

Andreae, M. O. and Merlet, P.: Emission of trace gases and aerosols from biomass burning, Global Biogeochem. Cy., 15, 955-966, https://doi.org/10.1029/2000GB001382, 2001.

Andreae, M. O.: Emission of trace gases and aerosols from biomass burning – an updated assessment, Atmos. Chem. Phys., 19, 8523-8546, https://doi.org/10.5194/acp-19-8523-2019, 2019.  Biomass burning emission factors https://edmond.mpdl.mpg.de/imeji/collection/op2vVE8m0us_gcGC, ver 14 Apr 2021.

Baray, S., Darlington, A., Gordon, M., Hayden, K. L., Leithead, A., Li, S. M., Liu, P. S. K., Mittermeier, R. L., Moussa, S. G., O'Brien, J., Staebler, R., Wolde, M., Worthy, D., and McLaren, R.: Quantification of methane sources in the Athabasca Oil Sands Region of Alberta by aircraft mass balance, Atmos. Chem. Phys., 18, 7361-7378, https://doi.org/10.5194/acp-18-7361-2018, 2018.

Baumgardner, D., Kok, G., and Raga, G.: Warming of the Arctic lower stratosphere by light absorbing particles, Geophys. Res. Lett., 31, L06117, https://doi.org/10.1029/2003GL018883, 2004.

Bertschi, I., Yokelson, R. J., Ward, D. E., Babbitt, R. E., Susott, R. A., Goode, J. G., and Hao, W. M.: Trace gas and particle emissions from fires in large diameter and belowground biomass fuels, J. Geophys. Res.-Atmos, 108, 8472, https://doi.org/10.1029/2002JD002100, 2003.

Cai, Y., Montague, D. C., Mooiweer-Bryan, W., and Deshler, T.: Performance characteristics of the ultra high sensitivity aerosol spectrometer for particles between 55 and 800nm: Laboratory and field studies, J. Aerosol Sci., 39, 759-769, https://doi.org/10.1016/j.jaerosci.2008.04.007, 2008.

Clyne, M. A. A., Thrush, B. A., and Wayne, R. P.: Kinetics of the chemiluminescent reaction between nitric oxide and ozone, Trans. Faraday Soc., 60, 359-370, https://doi.org/10.1039/TF9646000359, 1964.

Cole, A. S., Steffen, A., Eckley, C. S., Narayan, J., Pilote, M., Tordon, R., Graydon, J. A., St. Louis, V. L., Xu, X., and Branfireun, B. A.: A survey of mercury in air and precipitation across Canada: Patterns and trends, Atmosphere, 5, 635-668, https://doi.org/10.3390/atmos5030635, 2014.

DeCarlo, P. F., Dunlea, E. J., Kimmel, J. R., Aiken, A. C., Sueper, D., Crounse, J., Wennberg, P. O., Emmons, L., Shinozuka, Y., Clarke, A., Zhou, J., Tomlinson, J., Collins, D. R., Knapp, D., Weinheimer, A. J., Montzka, D. D., Campos, T., and Jimenez, J. L.: Fast airborne aerosol size and chemistry measurements above Mexico City and Central Mexico during the MILAGRO campaign, Atmos. Chem. Phys., 8, 4027-4048, https://doi.org/10.5194/acp-8-4027-2008, 2008.

Ditto, J. C., He, M., Hass-Mitchell, T. N., Moussa, S. G., Hayden, K., Li, S. M., Liggio, J., Leithead, A., Lee, P., Wheeler, M. J., Wentzell, J. J. B., and Gentner, D. R.: Atmospheric evolution of emissions from a boreal forest fire: the formation of highly functionalized oxygen-, nitrogen-, and sulfur-containing organic compounds, Atmos. Chem. Phys., 21, 255-267, https://doi.org/10.5194/acp-21-255-2021, 2021.

Drewnick, F., Schwab, J. J., Jayne, J. T., Canagaratna, M., Worsnop, D. R., and Demerjian, K. L.: Measurement of ambient aerosol composition during the PMTACS-NY 2001 using an aerosol mass spectrometer. Part I: Mass concentrations special issue of aerosol science and technology on findings from the Fine Particulate Matter Supersites Program, Aerosol Sci. Tech., 38, 92-103, https://doi.org/10.1080/02786820390229507, 2004.

Dunlea, E. J., DeCarlo, P. F., Aiken, A. C., Kimmel, J. R., Peltier, R. E., Weber, R. J., Tomlinson, J., Collins, D. R., Shinozuka, Y., McNaughton, C. S., Howell, S. G., Clarke, A. D., Emmons, L. K., Apel, E. C., Pfister, G. G., van Donkelaar, A., Martin, R. V., Millet, D. B., Heald, C. L., and Jimenez, J. L.: Evolution of Asian aerosols during transpacific transport in INTEX-B, Atmos. Chem. Phys., 9, 7257-7287, https://doi.org/10.5194/acp-9-7257-2009, 2009.

Fehsenfeld, F. C., Dickerson, R. R., Hübler, G., Luke, W. T., Nunnermacker, L. J., Williams, E. J., Roberts, J. M., Calvert, J. G., Curran, C. M., Delany, A. C., Eubank, C. S., Fahey, D. W., Fried, A., Gandrud, B. W., Langford, A. O., Murphy, P. C., Norton, R. B., Pickering, K. E., and Ridley, B. A.: A ground-based intercomparison of NO, NOx, and NOy measurement techniques, J. Geophys. Res.-Atmos, 92, 14710-14722, https://doi.org/10.1029/JD092iD12p14710, 1987.

Gordon, M., Li, S. M., Staebler, R., Darlington, A., Hayden, K., O'Brien, J., and Wolde, M.: Determining air pollutant emission rates based on mass balance using airborne measurement data over the Alberta oil sands operations, Atmos. Meas. Tech., 8, 3745-3765, https://doi.org/10.5194/amt-8-3745-2015, 2015.

Hatch, L. E., Yokelson, R. J., Stockwell, C. E., Veres, P. R., Simpson, I. J., Blake, D. R., Orlando, J. J., and Barsanti, K. C.: Multi-instrument comparison and compilation of non-methane organic gas emissions from biomass burning and implications for smoke-derived secondary organic aerosol precursors, Atmos. Chem. Phys., 17, 1471-1489, https://doi.org/10.5194/acp-17-1471-2017, 2017.

Hayden, K.L., Li, S.M. Makar, P., Liggio, J., Moussa, S.G., Akingunola, A., McLaren, R., Staebler, R.M., Darlington, A. , O’Brien, J., Zhang, J., Wolde, M., and Zhang, L.: New methodology shows short atmospheric lifetimes of oxidized sulfur and nitrogen due to dry deposition, Atmos. Chem. Phys., 21, 8377–8392, https://doi.org/10.5194/acp-21-8377-2021, 2021.

Jimenez, J. L., Jayne, J. T., Shi, Q., Kolb, C. E., Worsnop, D. R., Yourshaw, I., Seinfeld, J. H., Flagan, R. C., Zhang, X., Smith, K. A., Morris, J. W., and Davidovits, P.: Ambient aerosol sampling using the Aerodyne Aerosol Mass Spectrometer, J. Geophys. Res.-Atmos, 108, 8425, https://doi.org/10.1029/2001JD001213, 2003.

Khare, P., Marcotte, A., Sheu, R., Walsh, A. N., Ditto, J. C., and Gentner, D. R.: Advances in offline approaches for trace measurements of complex organic compound mixtures via soft ionization and high-resolution tandem mass spectrometry, J. Chromatogr. A, 1598, 163-174, https://doi.org/10.1016/j.chroma.2019.03.037, 2019.

Kleinman, L. I., Springston, S. R., Daum, P. H., Lee, Y. N., Nunnermacker, L. J., Senum, G. I., Wang, J., Weinstein-Lloyd, J., Alexander, M. L., Hubbe, J., Ortega, J., Canagaratna, M. R., and Jayne, J.: The time evolution of aerosol composition over the Mexico City plateau, Atmos. Chem. Phys., 8, 1559-1575, https://doi.org/10.5194/acp-8-1559-2008, 2008.

Koss, A. R., Sekimoto, K., Gilman, J. B., Selimovic, V., Coggon, M. M., Zarzana, K. J., Yuan, B., Lerner, B. M., Brown, S. S., Jimenez, J. L., Krechmer, J., Roberts, J. M., Warneke, C., Yokelson, R. J., and de Gouw, J.: Non-methane organic gas emissions from biomass burning: identification, quantification, and emission factors from PTR-ToF during the FIREX 2016 laboratory experiment, Atmos. Chem. Phys., 18, 3299-3319, https://doi.org/10.5194/acp-18-3299-2018, 2018.

Kupc, A., Williamson, C., Wagner, N. L., Richardson, M., and Brock, C. A.: Modification, calibration, and performance of the Ultra-High Sensitivity Aerosol Spectrometer for particle size distribution and volatility measurements during the Atmospheric Tomography Mission (ATom) airborne campaign, Atmos. Meas. Tech., 11, 369-383, https://doi.org/10.5194/amt-11-369-2018, 2018.

Lee, B. H., Lopez-Hilfiker, F. D., Mohr, C., Kurtén, T., Worsnop, D. R., and Thornton, J. A.: An Iodide-Adduct High-Resolution Time-of-Flight Chemical-Ionization Mass Spectrometer: Application to Atmospheric Inorganic and Organic Compounds, Environmental Science & Technology, 48, 6309-6317, 10.1021/es500362a, 2014.

Lee, B. H., Lopez-Hilfiker, F. D., Veres, P. R., McDuffie, E. E., Fibiger, D. L., Sparks, T. L., Ebben, C. J., Green, J. R., Schroder, J. C., Campuzano-Jost, P., Iyer, S., D'Ambro, E. L., Schobesberger, S., Brown, S. S., Wooldridge, P. J., Cohen, R. C., Fiddler, M. N., Bililign, S., Jimenez, J. L., Kurtén, T., Weinheimer, A. J., Jaegle, L., and Thornton, J. A.: Flight Deployment of a High-Resolution Time-of-Flight Chemical Ionization Mass Spectrometer: Observations of Reactive Halogen and Nitrogen Oxide Species, Journal of Geophysical Research: Atmospheres, 123, 7670-7686, https://doi.org/10.1029/2017JD028082, 2018.

Leifer, I., Melton, C. Tratt, D.M., Buckland, K.N., Clarisse, L., Coheur, P., Frash, J., Gupta, M., Johnson, P.D., Leen, J.B., Van Damme, M., Whitburn, S., and Yurganov, L.: Remote sensing and in situ measurements of methane and ammonia emissions from a megacity dairy complex: Chino, CA, Environ. Poll., 221, 37-51, https://doi.org/10.1016/j.envpol.2016.09.083, 2017.

Lerner, B. M., Gilman, J. B., Aikin, K. C., Atlas, E. L., Goldan, P. D., Graus, M., Hendershot, R., Isaacman-VanWertz, G. A., Koss, A., Kuster, W. C., Lueb, R. A., McLaughlin, R. J., Peischl, J., Sueper, D., Ryerson, T. B., Tokarek, T. W., Warneke, C., Yuan, B., and de Gouw, J. A.: An improved, automated whole air sampler and gas chromatography mass spectrometry analysis system for volatile organic compounds in the atmosphere, Atmos. Meas. Tech., 10, 291-313, https://doi.org/10.5194/amt-10-291-2017, 2017.

Li, S.-M., Leithead, A., Moussa, S. G., Liggio, J., Moran, M. D., Wang, D., Hayden, K., Darlington, A., Gordon, M., Staebler, R., Makar, P. A., Stroud, C. A., McLaren, R., Liu, P. S. K., O’Brien, J., Mittermeier, R. L., Zhang, J., Marson, G., Cober, S. G., Wolde, M., and Wentzell, J. J. B.: Differences between measured and reported volatile organic compound emissions from oil sands facilities in Alberta, Canada, P. Natl. Acad. Sci. USA, 114, E3756-E3765, https://doi.org/10.1073/pnas.1617862114, 2017.

Li, K., Liggio, J., Han, C., Liu, Q., Moussa, S. G., Lee, P., and Li, S.-M.: Understanding the impact of high-NOx conditions on the formation of secondary organic aerosol in the photooxidation of oil sand-related precursors, Environ. Sci. Technol., 53, 14420-14429, https://doi.org/10.1021/acs.est.9b05404, 2019.

Li, K., Wentzell, J. J. B., Liu, Q., Leithead, A., Moussa, S. G., Wheeler, M. J., Han, C., Lee, P., Li, S.-M., and Liggio, J.: Evolution of atmospheric total organic carbon from petrochemical mixtures, Environ. Sci. Technol., 55, 12841-12851, https://doi.org/10.1021/acs.est.1c02620, 2021.

Liggio, J., Stroud, C. A., Wentzell, J. J. B., Zhang, J., Sommers, J., Darlington, A., Liu, P. S. K., Moussa, S. G., Leithead, A., Hayden, K., Mittermeier, R. L., Staebler, R., Wolde, M., and Li, S.-M.: Quantifying the Primary Emissions and Photochemical Formation of Isocyanic Acid Downwind of Oil Sands Operations, Environmental Science & Technology, 51, 14462-14471, 10.1021/acs.est.7b04346, 2017.

McGee, T.K.: Evacuating First Nations during wildfires in Canada, Fire Safety Journ., 120, https://doi.org/10.1016/j.firesaf.2020.103120, 2020.

McLagan, D. S., Stupple, G. W., Darlington, A., Hayden, K., and Steffen, A.: Where there is smoke there is mercury: Assessing boreal forest fire mercury emissions using aircraft and highlighting uncertainties associated with upscaling emissions estimates, Atmos. Chem. Phys., 21, 5635-5653, https://doi.org/10.5194/acp-21-5635-2021, 2021.

Middlebrook, A. M., Bahreini, R., Jimenez, J. L., and Canagaratna, M. R.: Evaluation of composition-dependent collection efficiencies for the aerodyne aerosol mass spectrometer using field data, Aerosol Sci. Tech., 46, 258-271, https://doi.org/10.1080/02786826.2011.620041, 2012.

Mungall, E. L., Abbatt, J. P. D., Wentzell, J. J. B., Lee, A. K. Y., Thomas, J. L., Blais, M., Gosselin, M., Miller, L. A., Papakyriakou, T., Willis, M. D., and Liggio, J.: Microlayer source of oxygenated volatile organic compounds in the summertime marine Arctic boundary layer, Proceedings of the National Academy of Sciences of the United States of America, 114, 6203-6208, 10.1073/pnas.1620571114, 2017.

Neuman, J. A., Huey, L. G., Ryerson, T. B., and Fahey, D. W.: Study of Inlet Materials for Sampling Atmospheric Nitric Acid, Environmental Science & Technology, 33, 1133-1136, 10.1021/es980767f, 1999.

Penkett, S., Gilge, S., Plass-Duelmer, C., Galbally, I., Brough, N., Bottenheim, J., Flocke, F., Gerwig, H., Lee, J., Milton, M., Rohrer, F., Ryerson, T., Steinbacher, M., Torseth, K., and Wielgosz, R.: WMO/GAW expert workshop on global long-term measurements of nitrogen oxides and recommendations for GAW nitrogen oxides network, World Meteorological Organization, Hohenpeissenberg, Germany, 2011.

Permar, W., Wang, Q., Selimovic, V., Wielgasz, C., Yokelson, R. J., Hornbrook, R. S., Hills, A. J., Apel, E. C., Ku, I.-T., Zhou, Y., Sive, B. C., Sullivan, A. P., Collett Jr, J. L., Campos, T. L., Palm, B. B., Peng, Q., Thornton, J. A., Garofalo, L. A., Farmer, D. K., Kreidenweis, S. M., Levin, E. J. T., DeMott, P. J., Flocke, F., Fischer, E. V., and Hu, L.: Emissions of trace organic gases from western U.S. wildfires based on WE-CAN aircraft measurements, J. Geophys. Res.-Atmos, 126, e2020JD033838, https://doi.org/10.1029/2020JD033838, 2021.

Quinn, P. K., Bates, T. S., Coffman, D., Onasch, T. B., Worsnop, D., Baynard, T., de Gouw, J. A., Goldan, P. D., Kuster, W. C., Williams, E., Roberts, J. M., Lerner, B., Stohl, A., Pettersson, A., and Lovejoy, E. R.: Impacts of sources and aging on submicrometer aerosol properties in the marine boundary layer across the Gulf of Maine, J. Geophys. Res.-Atmos, 111, D23S36, https://doi.org/10.1029/2006JD007582, 2006.

Reid, J. S., and Hobbs, P. V.: Physical and optical properties of young smoke from individual biomass fires in Brazil, J. Geophys. Res.-Atmos., 103, 32013-32030, 10.1029/98jd00159, 1998.

Reid, J. S., Koppmann, R., Eck, T. F., and Eleuterio, D. P.: A review of biomass burning emissions part II: intensive physical properties of biomass burning particles, Atmos. Chem. Phys., 5, 799-825, 10.5194/acp-5-799-2005, 2005.

Ridley, B. A. and Grahek, F. E.: A small, low flow, high sensitivity reaction vessel for NO chemiluminescence detectors, J. Atmos. Ocean. Tech., 7, 307-311, https://doi.org/10.1175/1520-0426(1990)007<0307:Aslfhs>2.0.Co;2, 1990.

Roberts, J. M., Veres, P., Warneke, C., Neuman, J. A., Washenfelder, R. A., Brown, S. S., Baasandorj, M., Burkholder, J. B., Burling, I. R., Johnson, T. J., Yokelson, R. J., and De Gouw, J.: Measurement of HONO, HNCO, and other inorganic acids by negative-ion proton-transfer chemical-ionization mass spectrometry (NI-PT-CIMS): Application to biomass burning emissions, Atmospheric Measurement Techniques, 3, 981-990, 10.5194/amt-3-981-2010.

Schwarz, J. P., Gao, R. S., Fahey, D. W., Thomson, D. S., Watts, L. A., Wilson, J. C., Reeves, J. M., Darbeheshti, M., Baumgardner, D. G., Kok, G. L., Chung, S. H., Schulz, M., Hendricks, J., Lauer, A., Kärcher, B., Slowik, J. G., Rosenlof, K. H., Thompson, T. L., Langford, A. O., Loewenstein, M., and Aikin, K. C.: Single-particle measurements of midlatitude black carbon and light-scattering aerosols from the boundary layer to the lower stratosphere, J. Geophys. Res.-Atmos, 111, D16207, https://doi.org/10.1029/2006JD007076, 2006.

Sekimoto, K., Li, S.-M., Yuan, B., Koss, A., Coggon, M., Warneke, C., and de Gouw, J.: Calculation of the sensitivity of proton-transfer-reaction mass spectrometry (PTR-MS) for organic trace gases using molecular properties, Int. J. Mass Spectrom., 421, 71-94, https://doi.org/10.1016/j.ijms.2017.04.006, 2017.

Selimovic, V., Yokelson, R. J., McMeeking, G. R., and Coefield, S.: In situ measurements of trace gases, PM, and aerosol optical properties during the 2017 NW US wildfire smoke event, Atmos. Chem. Phys., 19, 3905-3926, https://doi.org/10.5194/acp-19-3905-2019, 2019.

Sheu, R., Marcotte, A., Khare, P., Charan, S., Ditto, J. C., and Gentner, D. R.: Advances in offline approaches for chemically speciated measurements of trace gas-phase organic compounds via adsorbent tubes in an integrated sampling-to-analysis system, J. Chromatogr. A, 1575, 80-90, https://doi.org/10.1016/j.chroma.2018.09.014, 2018.

Stecher III, H. A., Luther III, G. W., MacTaggart, D. L., Farwell, S. O., Crosley, D. R., Dorko, W. D., Goldan, P. D., Beltz, N., Krischke, U., Luke, W. T., Thornton, D. C., Talbot, R. W., Lefer, B. L., Scheuer, E. M., Benner, R. L., Wu, J., Saltzman, E. S., Gallagher, M. S., and Ferek, R. J.: Results of the Gas-Phase Sulfur Intercomparison Experiment (GASIE): Overview of experimental setup, results and general conclusions, J. Geophys. Res.-Atmos, 102, 16219-16236, https://doi.org/10.1029/97JD01362, 1997.

Stephens, M., Turner, N., and Sandberg, J.: Particle identification by laser-induced incandescence in a solid-state laser cavity, Appl. Optics, 42, 3726-3736, https://doi.org/10.1364/ao.42.003726, 2003.

Stockwell, C. E., Kupc, A., Witkowski, B., Talukdar, R. K., Liu, Y., Selimovic, V., Zarzana, K. J., Sekimoto, K., Warneke, C., Washenfelder, R. A., Yokelson, R. J., Middlebrook, A. M., and Roberts, J. M.: Characterization of a catalyst-based conversion technique to measure total particulate nitrogen and organic carbon and comparison to a particle mass measurement instrument, Atmos. Meas. Tech., 11, 2749-2768, 10.5194/amt-11-2749-2018, 2018.

Veres, P. R., and Roberts, J. M.: Development of a photochemical source for the production and calibration of acyl peroxynitrate compounds, Atmos. Meas. Tech., 8, 2225-2231, 10.5194/amt-8-2225-2015, 2015.

Veres, P., Roberts, J. M., Burling, I. R., Warneke, C., de Gouw, J., and Yokelson, R. J.: Measurements of gas-phase inorganic and organic acids from biomass fires by negative-ion proton-transfer chemical-ionization mass spectrometry, J. Geophys. Res.-Atmos, 115, D23302, https://doi.org/10.1029/2010JD014033, 2010.

Ward, D. E. and Radke, L. F.: Emissions measurements from vegetation fires: A comparative evaluation of methods and results, in: Fire in the Environment: The Ecological, Atmospheric, and Climatic Importance of Vegetation Fires. Dahlem Workshop Reports: Environmental Sciences Research Report 13, edited by: Crutzen, P. J., and Goldammer, J. G., John Wiley & Sons, Chischester, England, 53-76, 1993.

Williams, E. J., Baumann, K., Roberts, J. M., Bertman, S. B., Norton, R. B., Fehsenfeld, F. C., Springston, S. R., Nunnermacker, L. J., Newman, L., Olszyna, K., Meagher, J., Hartsell, B., Edgerton, E., Pearson, J. R., and Rodgers, M. O.: Intercomparison of ground-based NOy measurement techniques, J. Geophys. Res.-Atmos, 103, 22261-22280, https://doi.org/10.1029/98JD00074, 1998.