Modeling the formation and properties of traditional and non-traditional secondary organic aerosol: Problem formulation and application to aircraft exhaust

S. H. Jathar1,2, M. A. Miracolo2, A. A. Presto2,3, N. M. Donahue1,2,4, P. J. Adams1,2,5 and A. L. Robinson1,2,3

[1]{Engineering and Public Policy, Carnegie Mellon University, Pittsburgh PA}

[2]{Center for Atmospheric Particle Studies, Carnegie Mellon University, Pittsburgh PA}

[3]{Mechanical Engineering, Carnegie Mellon University, Pittsburgh PA}

[4]{Chemical Engineering, Carnegie Mellon University, Pittsburgh PA}

[5]{Civil and Environmental Engineering, Carnegie Mellon University, Pittsburgh PA}

Correspondence to: A. L. Robinson (alr@andrew.cmu.edu)

Abstract

We present a methodology to model secondary organic aerosol (SOA) formation from the photo-oxidation of unspeciated low-volatility organics (semi-volatile and intermediate volatile organic compounds) emitted by combustion systems. It is formulated using the volatility basis-set approach. Unspeciated low-volatility organics are classified by volatility and then allowed to react with the hydroxyl radical. The new methodology allows for larger reductions in volatility with each oxidation step than previous volatility basis set models, which is more consistent with those associated with the addition of common functional groups and similar to those used by traditional SOA models. The model is illustrated using data collected during two field campaigns that characterized the atmospheric evolution of dilute gas-turbine engine emissions using a smog chamber. In those experiments, photo-oxidation formed a significant amount of SOA, much of which could not be explained based on the emissions of traditional speciated precursors; we refer to unexplained SOA as non-traditional SOA (NT-SOA). The NT-SOA can be explained by emissions of unspeciated low-volatility organics measured using sorbents. We show that the parameterization proposed by Robinson et al. (2007) is unable to explain the timing of the NT-SOA formation in the aircraft experiments because it assumes a very modest reduction in volatility of the precursors with every oxidation reaction. In contrast the new Hybrid method better reproduces the NT-SOA formation. The NT-SOA yields estimated for the unspeciated low-volatility organic emissions in aircraft exhaust are similar to literature data for large n-alkanes and other low-volatility organics. The estimated yields vary with fuel composition (JP8 versus Fischer-Tropsch) and engine load (ground idle versus non-ground idle). The framework developed here is suitable for modeling SOA formation from emissions from other combustion systems.

# 1 Introduction

Atmospheric aerosols exert a large influence on climate and public health (Bernstein et al., 2004;IPCC, 2007). Secondary organic aerosol (SOA), defined as the organic particulate mass arising from the oxidation products of gas-phase organic species, accounts for a significant fraction of the submicron dry atmospheric aerosol mass (Zhang et al., 2007). Until recently, SOA formation was believed to be dominated by the first-generation oxidation products of high-flux volatile organic compounds (VOC) such as terpenes and single-ring aromatics. SOA formed from speciated VOCs is defined as traditional SOA (T-SOA) and is explicitly accounted for in chemical transport models. However, these models systematically under-predict organic aerosol levels (Heald et al., 2005;Vutukuru et al., 2006;Johnson et al., 2006;Morris et al., 2006;Dzepina et al., 2009;Dzepina et al., 2010), especially during photo-chemically active periods.

Recent laboratory and field studies show that combustion emissions when photo-oxidized form substantial SOA mass, greatly in excess of what can be explained by T-SOA models (Robinson et al., 2007;Grieshop et al., 2009;Hodzic et al., 2010;Miracolo et al., 2011;Miracolo et al., 2012). Robinson et al. (2007) proposed that a significant fraction of the unexplained SOA stemmed from the oxidation of unspeciated low-volatility organics, i.e. semi-volatile and intermediate volatility organic compounds (SVOC and IVOC). SVOCs refer to organic species that have an effective saturation concentration (C*) between 1 and 103 µg m-3 and IVOCs refer to species that have C* values between 104 and 107 µg m-3. S/IVOCs are co-emitted by combustion sources but are less volatile than VOC. However, these emissions are often not included in models because the vast majority of them cannot be speciated, they do not contribute significantly to ozone formation, and their measurement requires difficult-to-use sorbents. Fundamentally, S/IVOCs form SOA in the same manner as VOCs; gas-phase oxidation adds functional groups to the precursor molecule, creating lower volatility (vapor pressure) products that condense into the particle phase. The lower initial volatility of S/IVOCs mean that they can have higher SOA yields than VOCs (Lim and Ziemann, 2009b;Presto et al., 2010). SOA formed from S/IVOC vapors is denoted as non-traditional SOA (NT-SOA).

A key attribute of S/IVOC vapors is that the vast majority of their mass cannot be speciated by traditional one-dimensional gas-chromatography-based analytical techniques (Schauer et al., 1999, 2002). Instead they are classified as an unresolved complex mixture (UCM) that is thought to be dominated by co-eluting branched and cyclic alkanes (Robinson et al., 2007;Isaacman et al., 2012;Robinson et al., 2010;Schauer et al., 1999). The problem is fundamentally caused by the number of isomers growing exponentially with carbon number (Goldstein and Galbally, 2007). Since the molecular identity of the vast majority of S/IVOC mass cannot be ascertained, SOA formation from these compounds cannot be investigated or modeled in the same manner as traditional speciated SOA precursors (benzene, alpha-pinene, et al.). Instead, NT-SOA models have been based on the volatility of the emissions and a volatility-based oxidation mechanism (Robinson et al., 2007;Dzepina et al., 2009;Murphy and Pandis, 2009;Jathar et al., 2011).

Robinson et al. (2007) proposed a method (Robinson-2007) for NT-SOA formation in which volatility classified but unspeciated S/IVOC vapors react with the hydroxyl radical (OH) to form products that were one order of magnitude lower in volatility than their precursor. Pye and Seinfeld (2010) proposed a single-step mechanism for unspeciated SVOCs where the products of oxidation were two orders of magnitude lower in volatility than the precursor; they used SOA-yield data for naphthalene as a surrogate for all unspeciated IVOCs. Both methods have been implemented in plume, regional and global chemical transport models and help close large gaps between observed and predicted SOA concentrations (Shrivastava et al., 2008;Tsimpidi et al., 2009;Dzepina et al., 2010;Pye and Seinfeld, 2010;Jathar et al., 2011) and properties (Jathar et al., 2011;Shrivastava et al., 2008;Dzepina et al., 2009).

There are several shortcomings with existing methods to model NT-SOA formation. First, the parameters for those methods are based on very limited experimental data. In particular, the methods are not based on explicit fitting of experimental data like what is done for traditional SOA. For example, Shrivastava et al. (2008) showed that the parameters used in Robinson et al. (2007) reasonably predict the measured SOA formation from diesel exhaust; however, these parameters have been used to model all emissions (fossil fuel, bio fuel and biomass burning) (Shrivastava et al., 2008;Jathar et al., 2011). Pye and Seinfeld (2010) used naphthalene as a surrogate for all unspeciated IVOCs even though these are thought to be mainly branched and cyclic alkanes (Robinson et al., 2007;Isaacman et al., 2012;Robinson et al., 2010;Schauer et al., 1999). Second, existing mechanisms assume that each oxidation reaction reduces the volatility of the precursor by one to two orders of magnitude, which is less than the reduction seen with the addition of common functional groups (e.g. acids, nitrates, or carbonyls) (Kroll and Seinfeld, 2008). For VOCs such as benzene (Ng et al., 2007), isoprene (Ng et al., 2006) and cyclooctane (Tkacik et al., 2012), the particle-phase oxidation products are almost 5 to 8 orders of magnitude lower in volatility than the precursor. Third, the unspeciated IVOC emissions were not directly measured. For the Robinson-2007 method they were estimated by scaling POA emissions based on the work of Schauer et al. (1999, 2001, 2002); for the Pye and Seinfeld (2010) method they were estimated by scaling naphthalene emissions.

In this paper, we present a new method (Hybrid method) to represent NT-SOA formation from unspeciated S/IVOC vapors. First, we present the theoretical framework which is based on the volatility basis set approach (Donahue et al., 2006) and the work of Pankow (1994) and Odum et al. (1996). Next, to illustrate the new approach, it is applied to SOA data from smog chamber experiments conducted using dilute aircraft exhaust. A limitation of smog chamber experiments is their limited oxidant exposure; they only capture, at most, the atmospheric evolution of the first generation or two of oxidation of the precursors and their immediate products. Therefore, the data provide little constraint on the potential role of multi-generational oxidative aging. The Hybrid method is intended to be a general approach to parameterize NT-SOA formation from combustion sources. Here, aircraft exhaust is only used as an illustrative example.

# 2 SOA model formulation

- The modeling of both T-SOA and NT-SOA is commonly based on the approach of Pankow (1994) and Odum et al. (1996), which parameterizes smog chamber SOA data using a set of semi-volatile surrogate products. The amount of SOA is defined by the gas-particle partitioning of these surrogate products. While Odum et al. (1996) represented SOA with two surrogate products, more recently, researchers (Hildebrandt et al., 2009;Shakya and Griffin, 2010) have used four or more surrogates expressed using the volatility basis set (VBS) (Donahue et al., 2006). The VBS (Donahue et al., 2006) separates low-volatility organics into logarithmically spaced bins of effective saturation concentration (C*) between 0.01 to 107 µg m−3 at 298 K. C* (inverse of the Pankow-type partitioning coefﬁcient, Kp) is proportional to the saturation vapor pressure; it is a semi-empirical property that describes the gas-particle partitioning of an organic mixture (Pankow, 1994). The gas-particle partitioning of the set of surrogate products is calculated using absorptive partitioning theory:

- zi=1+CiCOA−1COA=i=1Nzi´Mi|g+p (1)

where, ζi is the fraction of mass in volatility bin ‘i’ in the particulate phase, Ci* is the effective saturation concentration of bin ‘i' in µg m−3, COA is the total particulate OA concentration in µg m−3, Mi|g+p is the total organic concentration (gas+particle) in bin ‘i’ in µg m−3 and N is the number of basis set bins. The VBS is used to track the concentration of all low-volatility organics (SOA and S/IVOC emissions). Although both the SOA formation and S/IVOC emissions can be tracked using a single basis set, for this work we use three separate basis sets to separately track different types of material. One VBS tracks the traditional semivolatile products formed from the oxidation of traditional speciated SOA precursors. A second tracks the fresh, un-oxidized S/IVOC emissions and a third tracks the oxidation products from the S/IVOC emissions.

T-SOA has traditionally been modeled using a distribution of first-generation, non-reactive surrogate products that were much lower in volatility than their precursor. More recently, these models have been extended to include multi-generational oxidation of the first-generation products (Lane et al., 2008;Farina et al., 2010;Murphy and Pandis, 2009). Previous work has modeled NT-SOA formation from unspeciated S/IVOCs with a simple, volatility-based multi-generational oxidation scheme (Robinson-2007) (Robinson et al., 2007;Shrivastava et al., 2008;Jathar et al., 2011). However, there are two potential shortcomings with this approach. First, the Robinson-2007 parameterization assumes that each oxidation reaction only reduces the volatility of the precursor by one order of magnitude. However, oxidation reactions form a variety of products with different volatilities; for example the addition of a single carbonyl, alcohol, nitrate or acid group creates a product with a volatility approximately 1, 3, 3 or 4 orders of magnitude lower than the precursor (Kroll and Seinfeld, 2008). Therefore, a more realistic NT-SOA parameterization would distribute the products over a set of volatility bins, with some of the bins having much lower volatility than the precursor species (similar to T-SOA models). Second, the Robinson-2007 parameterization assumes the same reduction in volatility for each generation of oxidation. However, recent experiments indicate that the reduction in volatility due to oxidation reactions changes as the molecules become more oxygenated and fragmentation (carbon-carbon scission) becomes important (Chacon-Madrid et al., 2010;Chacon-Madrid and Donahue, 2011;Kroll et al., 2011). Therefore, a more realistic NT-SOA parameterization would use a different mechanism for later generation products.

To address these shortcomings, we propose that the first generation of NT-SOA production from the oxidation of S/IVOC be treated similar to T-SOA (with precursor specific parameters) and that multi-generational oxidation be treated the same for all SOA. We call this the Hybrid approach, which enables a single, unified framework to be used to model both T-SOA and NT-SOA. We first describe that framework and then its application to develop parameterizations for NT-SOA formation.

The framework can be represented using the following equations:

d[Xj]dt=-kOx,Xj[Ox][Xj] (2)

d[Mi|g+p]dt=jαi,jkOx,Xj[Ox][Xj]first−generationproducts+kNβi,kkOx,Mk[Ox][Mk|g]production−kOx,Mi[Ox][Mi|g]lossmulti−generationaloxidation (3)

- Equation (2) represents the first-generation oxidation of SOA precursors (speciated VOC or S/IVOC) where kOx,Xj is the reaction rate between the oxidant [Ox] and SOA precursor [Xj]. The index j indicates different precursors, either speciated VOC precursors or volatility bins of the S/IVOC distribution. Equation (3) tracks the secondary organic material in VBS bin ‘i’. Mi|g+p is the total gas+particle organic mass in the ‘i’th bin of the VBS; its gas-particle partitioning is calculated using equation (1). The first term in equation (3) represents the first-generation products formed in ‘i’th bin as a result of the precursor oxidation where αi,j is the mass yield for the first-generation oxidation reaction. The second and third terms in equation (3) account for the continued evolution of material in the VBS due to multi-generational oxidation where we assume that only vapors in the VBS (M|g) react. k,i is the mass yield from multi-generational oxidation reactions in bin ‘k’ and kOx,M is the oxidation rate of vapors in the VBS.

To interpret smog chamber data, the framework (equations 1-3) is implemented in a box model that is comprised of two modules: a T-SOA and a NT-SOA module, both of which are described below. The T-SOA module is based on a standard SOA model formulated using the VBS (Pankow, 1994;Odum et al., 1996;Donahue et al., 2006); it uses the speciated VOC emissions and oxidant data to predict the amount of T-SOA that is formed. In the NT-SOA module, the amount of NT-SOA formed is first estimated by subtracting off the predicted T-SOA from the measured SOA. Then, the parameters in equations (1-3) are determined by fitting the NT-SOA data.

Defining the NT-SOA by difference effectively assumes that the T-SOA module is correct. However, published yields for T-SOA precursors (e.g. toluene) vary by more than a factor of two (Ng et al., 2007;Lane et al., 2008;Hildebrandt et al., 2009). As discussed below, the T-SOA model used for this work is based on upper end of the published data and therefore the difference approach may systematically underestimate the amount of NT-SOA.

- Interpreting smog chamber data with the NT-SOA model (equation (3)) requires differentiating between first and later generation oxidation products. In the context of this paper, the term “first generation” refers to the first set of stable products that arise after a series of oxidation reactions (Lim and Ziemann, 2005). The term “multi-generation” refers to the continued oxidation of the stable first-generation products. However, cleanly differentiating between the first and later generations of products is difficult in photo-oxidation experiments. In this paper, we empirically differentiate between first and later generation of oxidation production by assuming that the SOA formed during a typical 3 to 4 hour smog chamber experiment is dominated by the first generation of oxidation. This seems like a reasonably assumption because these experiments typically have aggregate hydroxyl radical exposures of around 107 molecules hr cm-3, which creates about one e-fold of oxidation for precursors with typical S/IVOC reaction rate constants (Atkinson and Arey, 2003). This assumption breaks down in systems in which later generation products are significantly more reactive than the original precursors.

- Based on our definition of first-generation (inside the smog chamber) and multi-generational (beyond the smog chamber) oxidation, theoretically we do not need to run the multi-generational oxidation mechanism while modeling SOA in the smog chamber. But doing so would incorrectly imply that the oxidation products formed in the smog chamber were non-reactive. Hence, for the sake of completeness (but at the cost of not being true to our definitions), we let the oxidation products that are formed in the smog chamber to undergo multi-generational oxidation.

# 2.1 Traditional SOA (T-SOA)

Previously, T-SOA has been defined as the non-reactive, stable oxidation product of traditional SOA precursors. In this work, we define T-SOA as the first and multi-generational SOA mass arising through the oxidation of speciated VOC precursors. To simulate T-SOA formation, Xj in equation (2) represents an individual precursor (e.g. benzene, toluene, n-dodecane, or cyclohexane) and OH is assumed to be the only oxidant. We use SAPRC lumping and mass-yields (αi,j in equation 2) proposed by Murphy and Pandis (2010) for all the speciated VOC precursors listed in Table 2. The Murphy and Pandis (2010) mass yields are at the high end of those reported in the literature; therefore the T-SOA prediction is an upper bound estimate, which, in turn, results in a lower bound estimate for NT-SOA. The lumping and parameters (kOx,Xj and αi,j) for the T-SOA model are provided in Tables S.1 and S.2 (supplementary material). Figure 1(a) shows a schematic for the T-SOA model.

To treat multi-generational oxidation of T-SOA, we use the parameterizations recently applied to anthropogenic SOA in regional and global models (Murphy and Pandis, 2009;Murphy and Pandis, 2010;Farina et al., 2010;Jathar et al., 2011). Gas-phase mass of the T-SOA products reacts with the OH radical (kOx,M = 1 x 10-11 cm3 molecules-1 s-1) to form products that are one order of magnitude lower in volatility than the precursor or shifted by one C* bin relative to the precursor. To account for the addition of oxygen, 7.5% of the precursor’s mass is added to the product. Hence, for T-SOA, the βi,k in equation (3) takes the form:

βi,k=+1.075ifk=i+1;0otherwise (4)

# 2.2 Non-traditional SOA (NT-SOA)

NT-SOA is defined as the SOA mass formed through the oxidation of unspeciated S/IVOC vapors. It is calculated experimentally as the difference between the measured SOA and the predicted T-SOA. In this section, we present two different approaches to parameterize the NT-SOA formation using the VBS framework (equations (1-3)). The methods differ in whether and how they account for first-generation oxidation and ongoing multi-generational oxidation (see Figure 1(b) and (c)).

- Robinson-2007 method

- Robinson et al. (2007) proposed a simple method to model NT-SOA formation, which uses a single oxidation kernel for all S/IVOC oxidation reactions. This method omits a detailed description of the volatility distribution of first-generation products and instead uses the same, simple, multi-generational oxidation scheme for every reaction. The method is shown schematically in Figure 1(b).

- The simplest way to implement this scheme is to place the volatility-resolved unspeciated S/IVOC precursor mass (Xj) directly into the corresponding VBS (Mi), which eliminates equation (2) and the first term in equation (3). Similar to the treatment of multi-generational oxidation for T-SOA (Lane et al., 2008), any gas-phase mass in the VBS reacts with the OH radical to form a product that is in a lower volatility bin than its precursor. For NT-SOA (Robinson-2007), βi,k takes the form:

βi,k=+(1+foxy)ifk=i+q;0otherwise (5)

where, q is the shift in volatility for the product and foxy is the fraction of oxygen added to the product per reaction.

To simulate NT-SOA formation using the Robinson-2007 method, one must define kOx,M , foxy and q. Robinson et al. (2007) and Shrivastava et al. (2008) used a kOx,M of 4 x 10-11 cm3 molecules-1 s-1, a foxy of 0.075 and a q of 1 based on SOA data for diesel exhaust. Grieshop et al. (2009a) proposed a kOH,M of 4 x 10-11 cm3 molecules-1 s-1, a foxy of 0.40 a q of 2 based on SOA data for dilute woodsmoke. Dzepina et al. (2009, 2010) and Hodzic et al. (2010) have applied these parameterizations to simulate SOA formation over Mexico City.

In this manuscript, we evaluate the previously proposed sets of kOH,M, foxy and q values.  We also fit the NT-SOA data to determine an optimum set of values for these parameters for the Robinson-2007 method.

- Hybrid method

The Hybrid method is similar to the previously discussed T-SOA model. It is shown schematically in Figure 1(c). The first generation of oxidation of S/IVOC vapors and subsequent NT-SOA formation is parameterized by fitting equations (1-3) to smog chamber data. A generic multi-generational aging scheme is then used for subsequent generations of oxidation. This allows for a more physically realistic treatment of the first-generation oxidation that better represents known effects of oxidation on volatility.

In the Hybrid method, for the first generation of oxidation, the volatility-resolved unspeciated S/IVOC emissions are treated as precursors (Xj) in equation (2) and their vapors are assumed to react only with the OH radical. We assume that kOx,Xj is 4 x 10-11 cm3 molecules-1 s-1 for vapors with a C* < 104 µg m-3 and 3x10-11 cm3 molecules-1 s-1 for vapors with C* ≥ 104 µg m-3 based on reactivity data for alkanes in these volatility ranges (Atkinson and Arey, 2003).

To determine the mass-yield matrix (αi,j in equation 3), we assume that each precursor undergoes several oxidation reactions in the smog chamber before it forms a stable set of products. Hence, we expect the particle-phase oxidation products to be much less volatile compared to its precursor. For a VOC like toluene (C* ~ 1.6 x 108 µg m-3), Hildebrandt et al. (2009) observed that the products of oxidation were centered around the C*=100 and C*=1000 µg m-3 bins, i.e. most of the products were 5 to 6 orders of magnitude lower than the precursor (toluene). Tkacik et al. (2012) observed that most of the measurable photo-oxidation products of cycloalkanes and branched alkanes were 3 to 6 orders of magnitude lower in volatility than their precursor. Therefore, we assume that in the Hybrid method, a precursor is allowed to form oxidation products that are 3 to 6 orders of magnitude lower in volatility than the precursor.

The mass-yield matrix (αi,j in equation 3) for the Hybrid method is derived by fitting the NT-SOA data. Since there are ten precursors (C* = 0.01 to 107 µg m-3; Table 2) and each precursor’s products are fit across 4 VBS bins, the Hybrid method potentially requires 40 free parameters (many more than can be constrained with the data). Presto et al. (2010), following the work of Lim and Ziemann (2009), found that for n-alkanes, the addition of 2 carbon atoms shifted its corresponding SOA product distribution, on average, by one C* bin or one order of magnitude in C* space. Therefore, we assume the same product distribution arising from each S/IVOC precursor bin, but shifted in volatility space by one order of magnitude. This approach reduces the number of free parameters to four. For instance, if [a1 b1 c1 d1] represents the mass yield for the precursor C* = 106 µg m-3 across C* bins [1 10 100 1000] (µg m-3), the mass-yield matrix αi,j for all precursors would take the same form shifted by one bin.

For multi-generational oxidation, we use the same set of parameters used to model the multi-generational oxidation of T-SOA (equation 4).

# 3 Experimental data

3.1 Overview of experimental methods

The Hybrid model is illustrated using data from smog chamber experiments conducted on diluted emissions from two different gas-turbine aircraft engines. Here, we provide a brief overview of both datasets; further details can be found in Miracolo et al. (2011), Presto et al. (2011), Miracolo et al. (2012) and Drozd et al. (2012). The first study investigated SOA formation from diluted emissions from a CFM56-2B gas-turbine engine operating on Jet Propellant – 8 (JP8) fuel (Presto et al., 2011;Miracolo et al., 2011) at four different engine loads (4% - ground idle, 7% - taxiing, 30% - landing and 85% - takeoff). In the second study, experiments were conducted on dilute emissions from a T63 gas turboshaft engine operating on JP8, Fischer-Tropsch (FT) and JP8/FT 50:50 blend fuels at ground idle and cruise loads. On a mass-basis, JP8 consists of 53% straight/cyclic alkanes, 30% branched alkanes and 17% aromatics while FT consists of 88% branched alkanes and 12% cyclic alkanes. The experiments used in this work are listed in Table 1, including the naming convention used in the paper.

Briefly, the experiments involved collecting emissions from about 1-m downstream of the engine exit plane and then transferring them through a heated transfer line into a portable Teflon smog chamber. The emissions were diluted 50 to 200 times with clean (HEPA- and activated-carbon filter) air to achieve primary PM concentration levels in the chamber that were representative of those typically found roughly 100-m downstream of the engine exit plane (< 10 g m-3). To initiate photo-oxidation, the chamber was exposed to natural or artificial sunlight; a suite of instruments tracked the evolution of the gas- and particle-phase pollutants. Photo-oxidation increased the aerosol levels inside the chamber, in certain experiments to 100s of g m-3. The relative humidity in the chamber was low (< 18%).

3.2 Overview of PM and SOA data

Figure 2 compiles the primary (elemental carbon and primary organic aerosol or POA) and secondary PM (sulfate and SOA) data from the two field campaigns. Briefly, EC and POA emissions were based on quartz filter samples collected off of a dilution tunnel (Drozd et al., 2012;Presto et al., 2011). Sulfate and SOA were based on AMS and SMPS measurements made at the end of ~3 to 4 hrs of photo-oxidation inside the smog chamber (Miracolo et al., 2011;Miracolo et al., 2012). The chamber data were converted to a fuel basis using the background-corrected measured CO2 concentration. The sum of the measured primary PM emissions and secondary PM formation spans two orders of magnitude (60-3300 mg kg-fuel-1) and is a strong function of the engine type, engine load and fuel. These variations are discussed in detail in companion publications (Presto et al., 2011;Miracolo et al., 2011;Miracolo et al., 2012;Presto et al., 2012;Drozd et al., 2012); here the focus is on modeling the SOA formation measured in the smog chamber. Briefly, at the end of every experiment, the wall-loss corrected secondary PM formation exceeded the direct primary PM emissions, by as much as a factor of 75. SOA accounts for more than half of the secondary PM mass (remainder is sulfate) except for in the CFM56-JP8-takeoff and T63-FT-cruise experiments and more than three quarters of the PM mass in the ground-idle experiments. On average, the T63 engine had higher emissions and higher secondary PM formation than the CFM56 engine. Both the SOA formation and precursor emissions decrease substantially with increasing engine load, i.e. ground idle vs. takeoff and ground idle vs. cruise.

- 3.3 Measured SOA precursors

Simulating SOA formation requires detailed information on SOA precursor concentrations. Table 2 reports VOC and S/IVOC emissions data for the different experiments. The VOC data were measured using SUMMA canisters that were analyzed using a GC-MS (Presto et al., 2011). VOC measurements were only made for one of three CFM56-JP8-Ground Idle experiments; therefore, we assumed that the same VOC emission profile applies to the other two experiments. For the CFM56-JP8-Taxi and CFM56-JP8-Landing experiments, only a small number of VOC were measured (Presto et al., 2011) and therefore we estimated emissions of additional VOC using data from the APEX study (Wey et al., 2006). The VOC emissions at taxi were assumed to be 40% of those at ground idle and VOC emissions at landing were assumed to be the same as those at takeoff.

S/IVOC emissions were characterized by GC-MS analysis of quartz filter and Tenax TA sorbent tube samples (Presto et al., 2011). Formally, we define S/IVOC as the sum of (both speciated and unspeciated) emissions that have a C* lower than or equal to 107 µg m-3. Presto et al. (2011) speciated less than 10% of the S/IVOC emissions (similar to studies done with other sources (Schauer et al., 1999, 2002)) the remainder was reported as an unresolved complex mixture. To estimate the total mass of S/IVOC emissions, Presto et al. (2012) developed a calibration curve for the UCM mass with fuel and lubricating oil used by the aircraft. The emissions were then distributed into the VBS based on the GC elution time (Presto et al., 2012). Further, they found that the chromatogram for all ground-idle emissions appeared to peak near a C* of 106 µg m-3 implying that there were considerable emissions of species with a C* greater than 106 µg m-3 that could not be quantified but likely also contribute to SOA formation. To ensure the inclusion of all low volatility organics that are capable of forming SOA, we assume that the mass of emissions in the C*=107 µg m-3 bin equals the mass in the C*=106 µg m-3 bin for all the ground-idle experiments. Table 2 reports measured S/IVOC emissions as a function of C*.

Figure 3 plots the measured SOA and its precursors – S/IVOC and VOC – for the different experiments. The bar labeled VOCs in Figure 3 only includes SOA precursors based on the SAPRC classification. Apart from the T63-JP8-Cruise experiment, the measured SOA mass is smaller than the sum of the precursor (S/IVOC + VOC) emissions. The precursor mass should be larger than the SOA mass if the SOA is formed from gas-phase oxidation of organic vapors. The VOC data were of poor quality in the cruise experiments. We hypothesize that these precursors may be mostly oxygenated and therefore poorly detected by the hydrocarbon-focused techniques employed by Presto et al. (2012). The S/IVOC emissions, on average, are larger than the speciated SOA precursors and therefore likely to be very important SOA precursors. Most of the S/IVOC emissions are IVOC.

- 3.4 Oxidant concentrations

The vast majority of the SOA precursors in aircraft exhaust are saturated species (there are significant unsaturated light VOCs, which do not form SOA); therefore the SOA formation in the smog chamber experiments is largely driven by oxidation by the hydroxyl radical (OH) and not by ozone. OH concentrations were not directly measured but inferred from the measured decay of organic (e.g. toluene) and inorganic (e.g. SO2) species. The OH concentration varied with time; it was about 107 molecules cm-3 at the beginning of the experiment and dropped to 106 molecules cm-3 by the end. This time variation was accounted for in the modeling. For some experiments, we estimated the OH exposure only using high reactivity species (kOH > 10-11 cm3 molecules-1 s-1) to reduce uncertainties associated with any chamber leakage. Figure 4 shows the median OH exposure (orange cross) with the standard error of the mean (green bars) calculated for each experiment. The OH exposure ranges from 4 to almost 50 hours of atmospheric oxidation at a typical OH concentration of 106 molecules cm-3.

# 4 Results

- 4.1 T-SOA

Model predictions for T-SOA are compared to the measured SOA in Figure 5(a). Each point represents a time-averaged value over ~ 100 seconds from an individual experiment. The CFM56 and T63 data are presented in separate panels. The model predicts that aromatics are the most important T-SOA precursors. In order to quantify the model-measurement comparison, we calculate the fractional error:

FractionalError=1Ni=1NP−MP+M2 (7)

where P is the predicted SOA, M is the measured SOA mass and N is number of data points. The fractional error is calculated using all of the data. Fractional error values are listed in Figure 5(a). Except for the CFM56-JP8-Takeoff and T63-FT-Ground Idle experiments, the T-SOA module predicts less than half of the measured SOA. T-SOA explains most of the SOA measured during the CFM56-JP8-Takeoff and T63-FT-Ground Idle experiments probably because the SOA mass yields of Murphy and Pandis (2010) are at the high end of those reported in the literature. Figure S.1 plots the ratio of predicted T-SOA to the measured SOA. We hypothesize that the large unexplained SOA is a direct result of S/IVOC oxidation.

4.2 NT-SOA formed versus S/IVOC reacted

The NT-SOA is estimated by subtracting the T-SOA prediction from the measured SOA. Except for the T63-FT experiments, NT-SOA accounts for anywhere between 30 and 96% of the SOA measured in the chamber.

Before applying the NT-SOA models, we first examine the mass balance between the estimated NT-SOA and the estimated mass of reacted S/IVOC. For this calculation, we assume that the SVOC react with the OH radical with a reactivity of 4 x 10-11 cm3 molecules-1 s-1 and IVOC react with the OH radical with a reactivity of 3 x 10-11 cm3 molecules-1 s-1. To quantify the mass balance, we calculate an effective NT-SOA yield, which is defined as follows:

EffectiveNT−SOAYield=NT−SOAformedSIVOCreacted (8)

Figure 6 plots the effective NT-SOA yield as a function of the OA concentration (COA). There are several important points to make from the plot. First, from a mass balance perspective, the NT-SOA yields are reasonable (i.e. they are less than 1), which means that the amount of NT-SOA formed is less than the amount of S/IVOC reacted. Second, the effective NT-SOA yields are similar to published yield data for speciated IVOCs. For example, Figure 6 indicates that for the JP8 experiments, the effective NT-SOA yields fall between the measured yields for n-dodecane (C12) and n-tridecane (C13). This is not surprising since the UCM distribution of both the emissions and unburned fuel peak between C11 and C15 (Corporan et al., 2011;Presto et al., 2011). Finally in most experiments, the NT-SOA yields increase with increasing COA, implying that the NT-SOA is semi-volatile, similar to T-SOA formed in smog chamber experiments (Odum et al., 1996).

The effective yields plotted in Figure 6 also appear to depend on both engine load and fuel composition. First, the ground-idle experiments appear to have higher yields than non-ground-idle experiments. This could be due to differences in precursor composition; in other words, the ground-idle S/IVOC emissions are comprised of compounds that more efficiently produce SOA than non-ground-idle emissions. If true, then different NT-SOA parameterizations would need to be developed for different engine loads. Alternatively, the higher ground-idle experiment yields may be due to partitioning differences (ground-idle experiments were conducted at higher COA). Second, the NT-SOA yields for JP8-Ground Idle are higher than Blend-Ground Idle yields which are higher than FT-Ground Idle yields. Accounting for differences in COA, it could be argued that Blend-Ground Idle yields are an arithmetic average of the JP8-Ground Idle and FT-Ground Idle yields. Miracolo et al. (2012) showed that the differences in SOA formation between JP8 and FT could mostly be attributed to compositional differences in the fuels. FT is mainly comprised of branched alkanes, which have low SOA yields versus JP8 which contains similar volatility but higher SOA yield n-alkanes and aromatics. Therefore, different NT-SOA parameterizations may be needed for different fuel types.

- 4.3 Parameterizing NT-SOA formation

- In this section we develop parameterizations for NT-SOA formation by fitting the measured SOA production. The goal is to determine an optimum parameter-set for the Robinson-2007 (kOH,M, foxy and q) and Hybrid approaches (αi,j; equation 6).

- Robinson-2007 method

The Robinson-2007 method, when using the Robinson et al. (2007) and Grieshop et al. (2009) parameter sets, under-predicts the NT-SOA formed during ground-idle experiments but over-predicts it during non-ground-idle experiments. Therefore, we fit the NT-SOA data to find an optimum parameter set for the Robinson-2007 method. We considered a wide but realistic range of reaction rates (kOH), fraction of oxygen added to the product per reaction (foxy) and shift in volatility (q). For kOH, we use a range of 1 to 5 x 10-11 cm3 molecules-1 s-1 based on Atkinson and Arey (2003). For foxy, we use a range of 0.05 to 0.4, which corresponds to the addition of 1 to 5 oxygen atoms per oxidation reaction to a C15 alkane. For q, we use a value of either 1 or 2, which corresponds to 1 or 2 orders of magnitude change in the product volatility with each oxidation reaction. Within these ranges, the optimum set was determined by minimizing the fractional error (equation 7) between model predictions and measurements for each experiment.

For the ground-idle experiments (except for the FT-Ground Idle) an aggressive parameter-set (kOH = 3-5 x 10-11 cm3 molecules-1 s-1, foxy = 0.05-0.4, q = 2) is required to fit the SOA data. In comparison, but a more modest parameter-set (kOH = 1-3 x 10-11 cm3 molecules-1 s-1, foxy = 0.05-0.3, q = 1) is sufficient to describe the non-ground-idle SOA data. The results are illustrated in Figure 5(b), which plots model predictions using the Robinson-2007 method with the best fit for each experiment against the SOA measured in the chamber. Compared to predictions from the T-SOA model alone, the model predictions are improved for the CFM56 experiments but only slightly improved for the T63 experiments. The improvement is quantified by the fractional error values listed in Figure 5.

Although an optimum parameter set provides some improvement over the T-SOA model, the Robinson-2007 method cannot reproduce the temporal trend in the data. The problem is illustrated in Figure 7, which plots time series of measured SOA production. The measured production either varies linearly or rolls over with OH exposure in the chamber. However, the NT-SOA calculated using the Robinson-2007 method shows the opposite trend with little NT-SOA formed initially and significantly more is formed later. This effect is most clearly seen for the T63-Blend-Ground Idle case plotted in Figure 8. It occurs because the Robinson-2007 approach requires (given the volatility distribution of the aircraft emissions) several generations of oxidation (a lot of OH exposure) before a large fraction of the products have a C* low enough to partition into the particle phase. The problem is most severe in the ground-idle experiments where almost all of the emissions are IVOC (Table 2). The Robinson-2007 method works for the CFM56-JP8-Taxi and CFM56-JP8-Takeoff experiments primarily because a sizeable fraction of the emissions are in the lower C* bins (C* = 102-104 µg m-3; Table 2).

Estimates of the O:C ratio of the OA reveals additional problems with the Robinson-2007 method. The O:C of the POA was measured before the oxidation phase of the experiment (Miracolo et al. 2012). For T-SOA, we use the work of Chhabra et al. (2010) to assign the O:C for SOA formed from alkenes and aromatics and the work of Presto et al. (2010) to assign the O:C for SOA formed from alkanes. For NT-SOA, we calculate O:C by explicitly tracking the addition of oxygen per reaction (foxy). For a few of the experiments, the optimum parameter-set for the Robinson-2007 method predicts a very high O:C ratio (>0.8) of OA (much higher than that estimated from the aerosol mass spectrometer data). This occurs because precursors have to go through multiple generations of oxidation before they reach a low enough volatility to partition into the particle phase. A consequence of this is that a lot of oxygen is added, with the exact amount depending on the values of foxy and q. For example, for the optimized parameter-set for the T63-JP8-Ground Idle experiment (kOH = 5 x 10-11 cm3 molecules-1 s-1, foxy = 0.40, q = 2), the O:C of the product would be close to 1 after only two generations of oxidation versus 0.32 for the measured data. It is clear that the Robinson-2007 approach is unable to reproduce both the temporal dependence of NT-SOA and the O:C of OA.

- Hybrid method

For each experiment, we fit the NT-SOA data to determine a set of VBS yields (equation 6) for the Hybrid method. Figure 7 plots the time series of measured and predicted OA for each experiment, with the NT-SOA calculated using the best fit for the Hybrid method. The upper and lower bounds for the measured SOA are presented; they are based on the experimental uncertainty due to wall-losses (we have not accounted for uncertainty in the T-SOA model). The predicted contribution from the first generation of oxidation of S/IVOC is labeled ‘NTSOA (1st generation)’ and the contribution from multi-generational oxidation is labeled ‘NTSOA (aged)’. Figure 7 indicates that the multi-generational oxidation – as defined by equation (4) – contributes negligibly to the SOA mass over the range of oxidant exposures observed in these experiments. Scatter plots of the model versus measurements are shown in Figure 5(c). The Hybrid method more accurately describes the data better than the Robinson-2007 method with significantly lower fractional errors.

To compare the Hybrid method fits across different experiments, Figure 9 plots the NT-SOA yields for select VBS bins precursors as a function of COA for the JP8 experiments. The effective yield is defined as the SOA formed by each bin divided by the mass of S/IVOC precursor reacted from that bin. Figure 9(a) shows yields for the precursors in the 103 and 104 µg m-3 bins and 8(b) shows yields for precursors in the 105, 106 and 107 µg m-3 bins. For visual clarity, we have not plot yields for S/IVOC precursors that contribute less than 15% to the NT-SOA mass. The lower C* surrogates generally do not contribute much NT-SOA mass because their emissions are either low and/or because very little of their mass exists as vapors and is therefore available for oxidation.

For all of the ground-idle experiments (Figure 9(a)), irrespective of the field campaign, almost all of the NT-SOA is produced from the oxidation of IVOC (C* bins: 105,106 and 107 µg m-3). This is expected because 90% of the low-volatility organic emissions are IVOC which peak at C*=106 and 107 µg m-3. In contrast, for the non-ground-idle experiments (Figure 9(b)), the NT-SOA arises from less volatile S/IVOCs in the 103 and 104 µg m-3 bins, which contribute a larger fraction of the emissions at higher loads

Figure 9(a) shows that the estimated SOA yields for IVOC in the 105, 106 and 107 µg m-3 bins (symbols) are equal or higher than published yields for n-alkanes in the same C* range (dashed lines; C14 ~ 105 µg m-3, C12 ~ 106 µg m-3 and C10 ~ 107 µg m-3). We believe that the higher yields are due to unburned fuel being a large component of the ground-idle emissions (Presto et al., 2011).  JP8 fuel contains significant amounts of cycloalkanes and aromatics which have higher SOA yields than n-alkanes. In contrast, Figure 9(b) shows that the effective SOA yields for S/IVOC in the 103 and 104 µg m-3 bins (symbols) are lower than the published yields for n-alkanes (Presto et al., 2010) in the same C* range (dashed lines; C18 ~ 103 µg m-3 and C16 ~ 104 µg m-3). We hypothesize that S/IVOC emissions during non-ground-idle conditions are dominated by species that have lower SOA yields than similar volatility n-alkanes such as branched alkanes (Lim and Ziemann, 2009b, a, 2005;Tkacik et al., 2012) or carbonyls (Chacon-Madrid and Donahue, 2011;Chacon-Madrid et al., 2010).

Given the apparent differences between ground-idle and non-ground-idle experiments, we developed two different parameter sets (αi,j) for the NT-SOA model. Table 3 lists the mass yields for the ground-idle and non-ground-idle conditions. The information in Table 3 can be interpreted using the mass-yield matrix defined in equation (6).

# 5 Conclusions and discussion

Many combustion systems emit substantial amounts of S/IVOCs (Schauer et al., 2002, 1999, 2001) that cannot be speciated but are likely to be important SOA precursors. The SOA formed from these vapors is defined as NT-SOA. In this paper, we develop a Hybrid method based on the VBS, to model SOA formation from unspeciated S/IVOC vapors. Unspeciated S/IVOCs are classified by volatility and then allowed to react with the hydroxyl radical. The Hybrid method allows for a volatility distribution of products and for larger reductions in volatility with each oxidation step than previous parameterizations. The Hybrid method is consistent with the formation of oxidation products observed for traditional SOA species and the reduction in volatility calculated for the addition of common functional groups. Compared to previous parameterizations, the Hybrid method provides a more realistic representation of NT-SOA formation from S/IVOCs in the atmosphere.

We applied the Hybrid method to model SOA data from smog chamber experiments conducted on diluted aircraft exhaust. This analysis revealed that speciated VOCs accounted for less than half of the measured SOA in most experiments.  The following pieces of evidence suggest that unspeciated S/IVOC vapors are important SOA precursors.  First, Tenax measurements demonstrated substantial unspeciated S/IVOC emissions, exceeding those of traditional SOA precursors.  Estimated NT-SOA yields using these measured vapors and reasonable reaction rates are similar to chamber data for individual S/IVOC species.  The approach of Robinson et al. (2007) cannot reproduce the temporal evolution of SOA production. The fits using the Hybrid method were able to reproduce the temporal evolution of SOA production. And finally, we developed parameters for the Hybrid method to model NT-SOA from S/IVOCs emissions in aircraft exhaust.

In addition to varying with COA, the NT-SOA yields (efficiency of S/IVOCs to form NT-SOA) appeared to be a function of both the (a) fuel composition and (b) engine load. This is not surprising since both molecular structure in addition to volatility influences SOA yields (Lim and Ziemann, 2009b, a;Tkacik et al., 2012). The effective NT-SOA yield is highest for JP8 and lowest for FT while the 50:50 JP8:FT blend appears to be an average of JP8 and FT. JP8 consists mostly of straight/cyclic alkanes (53%) and aromatics (17%), which form more SOA than similar volatility branched alkanes that mostly constitute FT (88%) (Lim and Ziemann, 2009b;Presto et al., 2010;Tkacik et al., 2012;Lim and Ziemann, 2005;Corporan et al., 2011). The effective NT-SOA yields also appear to be higher for JP8 ground-idle emissions than for JP8 non-ground-idle emissions. We suspect that ground-idle emissions are comprised of unburned cycloalkanes and aromatic compounds found in the fuel while non-ground-idle emissions are comprised of low SOA-yield species such as branched alkanes and carbonyls (Chacon-Madrid and Donahue, 2011;Tkacik et al., 2012;Chacon-Madrid et al., 2010). Therefore, different NT-SOA parameterizations may be needed for different fuels and different engine loads.

Table 3 provides parameters for the Hybrid method to model NT-SOA from S/IVOC arising from aircraft emissions. Two sets of parameters are provided; one for ground-idle emissions and one for non-ground-idle emissions. They are suitable for use with the VBS framework in any box, plume, regional or global OA model using the emissions data listed in Table 2. Although these emissions were data are based on measurements made on two engines, they could be extended to other engines by using ratios with high-flux species like acetylene, propene or benzene.

They Hybrid method leverages volatility information of the emissions to model NT-SOA from unspeciated low-volatility organics. It is clear from smog chamber studies that SOA formation is a function not only of the volatility of the precursor but also its molecular structure (e.g. aromatics form more SOA than similar volatility alkanes).  Although some progress has been made (Isaacman et al., 2012), it may be many years before we are able to study the molecular structure of emissions and develop models that account for it. Until then, one can think of the volatility-based method developed in this work as an interim solution to model SOA from low-volatility organics.

In this work, we need two different sets of parameters to model NT-SOA from aircraft engines running JP8 and it seems probable that additional parameters may be needed for aircraft engines running on FT. This implies that volatility alone is insufficient in describing the potential of the exhaust to form SOA.  One potential consequence of this is that the Hybrid method may require each source would need to be treated separately, which could complicate its implementation. However, it is unclear if this will apply to emissions from other combustion sources (e.g. vehicle exhaust, biomass burning).  This is an area for future research.

5 Acknowledgements

Funding was provided by the U.S. Department of Defense Strategic Environmental Research and Development Program (SERDP) under project WP-1626. The views, opinions, and/or findings contained in this paper are those of the authors and should not be construed as an official position of any of the funding agency.

6 References

Atkinson, R., and Arey, J.: Atmospheric degradation of volatile organic compounds, Chemical Reviews, 103, 4605-4638, 2003.

Bernstein, J. A., Alexis, N., Barnes, C., Bernstein, I. L., Bernstein, J. A., Nel, A., Peden, D., Diaz-Sanchez, D., Tarlo, S. M., and Williams, P. B.: Health effects of air pollution, The Journal of Allergy and Clinical Immunology, 114, 1116-1123, 2004.

Chacon-Madrid, H., and Donahue, N.: Fragmentation vs. Functionalization: Chemical aging and organic aerosol formation, Atmos. Chem. Phys, 11, 10553-10563, doi:10.5194/acp-11-10553-2011 2011.

Chacon-Madrid, H. J., Presto, A. A., and Donahue, N. M.: Functionalization vs. Fragmentation: N-aldehyde oxidation mechanisms and secondary organic aerosol formation, Phys. Chem. Chem. Phys., 12, doi: 10.1039/c0cp00200c, 2010.

Corporan, E., Edwards, T., Shafer, L., DeWitt, M. J., Klingshirn, C., Zabarnick, S., West, Z., Striebich, R., Graham, J., and Klein, J.: Chemical, thermal stability, seal swell, and emissions studies of alternative jet fuels, Energy & Fuels, 25, 955-966, dx.doi.org/10.1021/ef101520v, 2011.

Donahue, N., Robinson, A., Stanier, C., and Pandis, S.: Coupled partitioning, dilution, and chemical aging of semivolatile organics, Environ. Sci. Technol, 40, 2635-2643, doi:10.1021/es052297c, 2006.

Drozd, G. T., Miracolo, M. A., Presto, A. A., Lipsky, E. M., Riemer, D. D., Corporan, E., and Robinson, A. L.: Particulate matter and organic vapor emissions from an in-use helicopter engine operating on petroleum and fischer-tropsch fuels, Energy & Fuels, 2012.

Dzepina, K., Volkamer, R., Madronich, S., Tulet, P., Ulbrich, I., Zhang, Q., Cappa, C., Ziemann, P., and Jimenez, J.: Evaluation of recently-proposed secondary organic aerosol models for a case study in mexico city, Atmospheric Chemistry and Physics, 9, 5681-5709, doi:10.5194/acp-9-5681-2009, 2009.

Dzepina, K., Cappa, C. D., Volkamer, R. M., Madronich, S., DeCarlo, P. F., Zaveri, R. A., and Jimenez, J. L.: Modeling the multiday evolution and aging of secondary organic aerosol during milagro 2006, Environmental Science & Technology, 45, 3496-3503, doi: 10.1021/es103186f, 2010.

Farina, S. C., Adams, P. J., and Pandis, S. N.: Modeling global secondary organic aerosol formation and processing with the volatility basis set: Implications for anthropogenic secondary organic aerosol, Journal of Geophysical Research, 115, D09202, doi:10.1029/2009JD013046, 2010.

Goldstein, A. H., and Galbally, I. E.: Known and unexplored organic constituents in the earth's atmosphere, Environmental Science & Technology, 41, 1514-1521, doi:10.1021/es072476p, 2007.

Grieshop, A., Donahue, N., and Robinson, A.: Laboratory investigation of photochemical oxidation of organic aerosol from wood fires 2: Analysis of aerosol mass spectrometer data, Atmospheric Chemistry and Physics, 9, 2227-2240, 2009.

Heald, C. L., Jacob, D. J., Park, R. J., Russell, L. M., Huebert, B. J., Seinfeld, J. H., Liao, H., and Weber, R. J.: A large organic aerosol source in the free troposphere missing from current models, Geophys. Res. Lett., 32, doi:10.1029/2005GL023831, 2005.

Hildebrandt, L., Donahue, N., and Pandis, S.: High formation of secondary organic aerosol from the photo-oxidation of toluene, Atmospheric Chemistry and Physics, 9, 2973-2986, doi:10.5194/acp-9-2973-2009, 2009.

Hodzic, A., Jimenez, J., Madronich, S., Canagaratna, M., DeCarlo, P., Kleinman, L., and Fast, J.: Modeling organic aerosols in a megacity: Potential contribution of semi-volatile and intermediate volatility primary organic compounds to secondary organic aerosol formation, Atmospheric Chemistry and Physics, 10, 5491-5514, doi:10.5194/acp-10-5491-2010, 2010.

IPCC, W.: Climate change 2007: The physical science basis, Summary for Policy Makers, Contribution of Working Group I to the Fourth Assessment Report of the Intergovernmental Panel on Climate Change, 2007.

Isaacman, G., Wilson, K. R., Chan, A. W. H., Worton, D. R., Kimmel, J. R., Nah, T., Hohaus, T., Gonin, M., Kroll, J. H., and Worsnop, D. R.: Improved resolution of hydrocarbon structures and constitutional isomers in complex mixtures using gas chromatography-vacuum ultraviolet-mass spectrometry (gc-vuv-ms), Analytical Chemistry, 84, 2335-2342, 2012.

Jathar, S., Farina, S., Robinson, A., and Adams, P.: The influence of semi-volatile and reactive primary emissions on the abundance and properties of global organic aerosol, Atmospheric Chemistry and Physics, 11, 7727-7746, doi:10.5194/acp-11-7727-2011 2011.

Johnson, D., Utembe, S. R., Jenkin, M. E., Derwent, R. G., Hayman, G. D., Alfarra, M. R., Coe, H., and McFiggans, G.: Simulating regional scale secondary organic aerosol formation during the torch 2003 campaign in the southern uk, Atmos. Chem. Phys., 6, 403-418, 2006.

Kroll, J. H., and Seinfeld, J. H.: Chemistry of secondary organic aerosol: Formation and evolution of low-volatility organics in the atmosphere, Atmospheric Environment, 42, 3593-3624, 2008.

Kroll, J. H., Donahue, N. M., Jimenez, J. L., Kessler, S. H., Canagaratna, M. R., Wilson, K. R., Altieri, K. E., Mazzoleni, L. R., Wozniak, A. S., Bluhm, H., Mysak, E. R., Smith, J. D., Kolb, C. E., and Worsnop, D. R.: Carbon oxidation state as a metric for describing the chemistry of atmospheric organic aerosol, Nature Chemistry, 3, 133-139, 2011.

Lane, T. E., Donahue, N. M., and Pandis, S. N.: Simulating secondary organic aerosol formation using the volatility basis-set approach in a chemical transport model, Atmospheric Environment, 42, 7439-7451, 2008.

Lim, Y. B., and Ziemann, P. J.: Products and mechanism of secondary organic aerosol formation from reactions of n-alkanes with oh radicals in the presence of no x, Environmental Science & Technology, 39, 9229-9236, 2005.

Lim, Y. B., and Ziemann, P. J.: Effects of molecular structure on aerosol yields from oh radical-initiated reactions of linear, branched, and cyclic alkanes in the presence of no x, Environmental Science & Technology, 43, 2328-2334, 2009a.

Lim, Y. B., and Ziemann, P. J.: Chemistry of secondary organic aerosol formation from oh radical-initiated reactions of linear, branched, and cyclic alkanes in the presence of no x, Aerosol Science and Technology, 43, 604-619, 2009b.

Miracolo, M., Hennigan, C., Ranjan, M., Nguyen, N., Gordon, T., Lipsky, E., Presto, A., Donahue, N., and Robinson, A.: Secondary aerosol formation from photochemical aging of aircraft exhaust in a smog chamber, Atmos. Chem. Phys, 11, 4135-4147, doi:10.5194/acp-11-4135-2011, 2011.

Miracolo, M. A., Drozd, G. T., Jathar, S., Presto, A. A., Lipsky, E., Corporan, E., and Robinson, A.: Fuel composition and secondary organic aerosol formation: Gas-turbine exhaust and alternative aviation fuels, Environmental Science & Technology, 46, 8493-8501, doi:10.1021/es300350c, 2012.

Morris, R. E., Koo, B., Guenther, A., Yarwood, G., McNally, D., Tesche, T. W., Tonnesen, G., Boylan, J., and Brewer, P.: Model sensitivity evaluation for organic carbon using two multi-pollutant air quality models that simulate regional haze in the southeastern united states, Atmos. Environ., 40, 4960-4972, 2006.

Murphy, B., and Pandis, S.: Simulating the formation of semivolatile primary and secondary organic aerosol in a regional chemical transport model., Environmental science & technology, 43, 4722-4728, doi:10.1021/es803168a, 2009.

Murphy, B. N., and Pandis, S. N.: Exploring summertime organic aerosol formation in the eastern united states using a regional-scale budget approach and ambient measurements, Journal of Geophysical Research, 115, D24216, doi:10.1029/2010JD014418, 2010.

Ng, N., Kroll, J., Chan, A., Chhabra, P., Flagan, R., and Seinfeld, J.: Secondary organic aerosol formation from m-xylene, toluene, and benzene, Atmos. Chem. Phys, 7, 3909-3922, doi:10.5194/acp-7-3909-2007, 2007.

Ng, N. L., Kroll, J. H., Keywood, M. D., Bahreini, R., Varutbangkul, V., Flagan, R. C., Seinfeld, J. H., Lee, A., and Goldstein, A. H.: Contribution of first-versus second-generation products to secondary organic aerosols formed in the oxidation of biogenic hydrocarbons, Environmental Science & Technology, 40, 2283-2297, 2006.

Odum, J. R., Hoffmann, T., Bowman, F., Collins, D., Flagan, R. C., and Seinfeld, J. H.: Gas/particle partitioning and secondary organic aerosol yields, Environmental Science & Technology, 30, 2580-2585, 1996.

Pankow, J. F.: An absorption model of gas/particle partitioning of organic compounds in the atmosphere, Atmospheric Environment, 28, 185-188, 1994.

Presto, A. A., Miracolo, M. A., Donahue, N. M., and Robinson, A. L.: Secondary organic aerosol formation from high-no x photo-oxidation of low volatility precursors: N-alkanes, Environmental Science & Technology, 44, 2029-2034, 2010.

Presto, A. A., Nguyen, N. T., Ranjan, M., Reeder, A. J., Lipsky, E. M., Hennigan, C. J., Miracolo, M. A., Riemer, D. D., and Robinson, A. L.: Fine particle and organic vapor emissions from staged tests of an in-use aircraft engine, Atmospheric Environment, 45, 3603-3612, 2011.

Presto, A. A., Hennigan, C. J., Nguyen, N. T., and Robinson, A. L.: Determination of volatility distributions of primary organic aerosol emissions from combustion systems using thermal desorption gas chromatography mass spectrometry, Aerosol Science and Technology, 46, 1129-1139, 2012.

Pye, H., and Seinfeld, J.: A global perspective on aerosol from low-volatility organic compounds, Atmos. Chem. Phys, 10, 4377-4401, doi:10.5194/acp-10-4377-2010, 2010.

Robinson, A. L., Donahue, N. M., Shrivastava, M. K., Weitkamp, E. A., Sage, A. M., Grieshop, A. P., Lane, T. E., Pierce, J. R., and Pandis, S. N.: Rethinking organic aerosols: Semivolatile emissions and photochemical aging, Science, 315, 1259-1262, 2007.

Robinson, A. L., Grieshop, A. P., Donahue, N. M., and Hunt, S. W.: Updating the conceptual model for fine particle mass emissions from combustion systems, Journal of the Air & Waste management association, 60, 1204-1222, 2010.

Schauer, J. J., Kleeman, M. J., Cass, G. R., and Simoneit, B. R. T.: Measurement of emissions from air pollution sources. 2. C1 through c30 organic compounds from medium duty diesel trucks, Environ. Sci. Technol, 33, 1578-1587, 1999.

Schauer, J. J., Kleeman, M. J., Cass, G. R., and Simoneit, B. R. T.: Measurement of emissions from air pollution sources. 3. C1- c29 organic compounds from fireplace combustion of wood, Environ. Sci. Technol, 35, 1716--1728, 2001.

Schauer, J. J., Kleeman, M. J., Cass, G. R., and Simoneit, B. R. T.: Measurement of emissions from air pollution sources. 5. C1- c32 organic compounds from gasoline-powered motor vehicles, Environ. Sci. Technol, 36, 1169-1180, 2002.

Shakya, K. M., and Griffin, R. J.: Secondary organic aerosol from photooxidation of polycyclic aromatic hydrocarbons, Environmental Science & Technology, 44, 8134-8139, doi:10.1021/es1019417, 2010.

Shrivastava, M. K., Lane, T. E., Donahue, N. M., Pandis, S. N., and Robinson, A. L.: Effects of gas particle partitioning and aging of primary emissions on urban and regional organic aerosol concentrations, Journal of Geophysical Research-Atmospheres, 113, D18301, doi:10.1029/2007JD009735, 2008.

Tkacik, D. S., Presto, A. A., Donahue, N. M., and Robinson, A. L.: Secondary organic aerosol formation from intermediate-volatility organic compounds: Cyclic, linear, and branched alkanes, Environmental Science & Technology, 2012.

Tsimpidi, A., Karydis, V., Zavala, M., Lei, W., Molina, L., Ulbrich, I., Jimenez, J., and Pandis, S.: Evaluation of the volatility basis-set approach for the simulation of organic aerosol formation in the mexico city metropolitan area, Atmos. Chem. Phys, 10, 525-546, doi:10.5194/acp-10-525-2010, 2009.

Vutukuru, S., Griffin, R. J., and Dabdub, D.: Simulation and analysis of secondary organic aerosol dynamics in the south coast air basin of california, J. Geophys. Res., 111, doi:10.1029/2005JD006139, 2006.

Wey, C., Anderson, B., Hudgins, C., Wey, C., Li-Jones, X., Winstead, E., Thornhill, L., Lobo, P., Hagen, D., and Whitefield, P.: Aircraft particle emissions experiment (apex), NASA TM-2006, 2006.

Zhang, Q., Jimenez, J. L., Canagaratna, M. R., Allan, J. D., Coe, H., Ulbrich, I., Alfarra, M. R., Takami, A., Middlebrook, A. M., Sun, Y. L., Dzepina, K., Dunlea, E., Docherty, K., DeCarlo, P. F., Salcedo, D., Onasch, T., Jayne, J. T., Miyoshi, T., Shimono, A., Hatakeyama, S., Takegawa, N., Kondo, Y., Schneider, J., Drewnick, F., Borrmann, S., Weimer, S., Demerjian, K., Williams, P., Bower, K., Bahreini, R., Cottrell, L., Griffin, R. J., Rautiainen, J., Sun, J. Y., Zhang, Y. M., and Worsnop, D. R.: Ubiquity and dominance of oxygenated species in organic aerosols in anthropogenically-influenced northern hemisphere midlatitudes, Geophys. Res. Lett, 34, L13801, doi:10.1029/2007GL029979, 2007.

7 Tables

Table 1: List of smog chamber experiments.

| No | Experiment Name | Engine | Fuel | Load | Reference |
| --- | --- | --- | --- | --- | --- |
| 1 | CFM56-JP8-Ground Idle(1) | CFM56-2B | JP8 | 4% | Presto et al. (2011, 2012), Miracolo et al. (2011) |
| 2 | CFM56-JP8-Ground Idle(2) | CFM56-2B | JP8 | 4% |  |
| 3 | CFM56-JP8-Ground Idle(3) | CFM56-2B | JP8 | 4% |  |
| 4 | CFM56-JP8-Taxi | CFM56-2B | JP8 | 7% |  |
| 5 | CFM56-JP8-Landing | CFM56-2B | JP8 | 30% |  |
| 6 | CFM56-JP8-Takeoff | CFM56-2B | JP8 | 85% |  |
| 7 | T63-JP8-Ground Idle | T63 | JP8 | Ground Idle | Miracolo et al. (2012), Drozd et al. (2012), Presto et al. (2012) |
| 8 | T63-FT-Ground Idle(1) | T63 | FT | Ground Idle |  |
| 9 | T63-FT-Ground Idle(2) | T63 | FT | Ground Idle |  |
| 10 | T63-Blend-Ground Idle | T63 | JP8:FT Blend | Ground Idle |  |
| 11 | T63-JP8-Cruise | T63 | JP8 | Cruise |  |
| 12 | T63-FT-Cruise | T63 | FT | Cruise |  |

Table 2: Emission factor (mg kg-fuel-1) for speciated VOCs and S/IVOCs for each engine, fuel and engine load (Miracolo et al., 2011;Miracolo et al., 2012;Presto et al., 2012).

| Type | Species | C* (µg m-3) | CFM56-JP8 |  |  |  | T63-JP8 |  | T63-FT |  | T63-Blend |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Ground Idle | Taxi | Landing | Takeoff | Ground Idle | Cruise | Ground Idle | Cruise | Ground Idle |
| VOC | 1-butene | 6.42E+09 | 194.6 | 58.4 | 2.2 | 2.2 | 401.1 | 1.2 | 159.1 | 1.5 | 292.7 |
|  | 1-heptene | 3.21E+08 | 61.5 | 18.5 | - | - | 0.1 | 0.0 | 6.3 | 0.0 | 11.6 |
|  | 1-hexene | 8.35E+08 | 81.1 | 24.3 | - | - | - | - | - | - | - |
|  | 1-methylcyclohexene | 3.90E+08 | 5.2 | 1.6 | - | - | - | - | - | - | - |
|  | 1-octene | 1.04E+08 | 5.9 | 1.8 | 1.2 | 1.2 | - | - | - | - | - |
|  | 1-pentene | 2.39E+09 | 91.2 | 27.4 | 10.8 | 10.8 | 81.7 | 0.0 | 69.8 | 0.0 | 0.0 |
|  | 1,2-butadiene | 2.71E+09 | 6.4 | 1.9 | - | - | 1.4 | 0.0 | 4.8 | 0.0 | 3.7 |
|  | 1,2-diethylbenzene | 7.04E+06 | 10.9 | 3.3 | 1.9 | 1.9 | - | - | - | - | - |
|  | 1,2,3-trimethylbenzene | 9.51E+06 | 47.0 | 14.1 | 1.7 | 1.7 | 4.3 | 0.0 | 10.8 | 0.0 | 32.7 |
|  | 1,2,4-trimethylbenzene | 1.30E+07 | 41.9 | 12.6 | 7.4 | 7.4 | 24.9 | 0.0 | 30.5 | 0.0 | 119.8 |
|  | 1,2,4,5-tetramethylbenzene | 1.12E+06 | 27.2 | 8.2 | - | - | - | - | - | - | - |
|  | 1,3-butadiene | 6.11E+09 | 230.3 | 69.1 | - | - | 391.3 | 2.8 | 76.4 | 1.4 | 0.0 |
|  | 1,3-diethylbenzene | 7.67E+06 | 10.2 | 3.1 | 1.8 | 1.8 | 14.6 | 1.2 | 180.9 | 0.0 | 125.4 |
|  | 1,3,5-trimethylbenzene | 1.54E+07 | 14.4 | 4.3 | 1.0 | 1.0 | 16.4 | 0.0 | 39.1 | 0.0 | 47.7 |
|  | 1,4-diethylbenzene | 7.13E+06 | 46.7 | 14.0 | 1.9 | 1.9 | 3.7 | 3.9 | 75.6 | 0.0 | 68.2 |
|  | 2-ethyltoluene | 1.59E+07 | 12.6 | 3.8 | 34.2 | 34.2 | 16.0 | 2.8 | 11.1 | 0.0 | 30.7 |
|  | 2-methyl-1-butene | 2.29E+09 | 30.3 | 9.1 | 1.0 | 1.0 | 52.5 | 0.0 | 81.1 | 0.0 | 26.6 |
|  | 2-methyl-1-pentene | 1.25E+09 | 10.6 | 3.2 | - | - | 5.2 | 0.0 | 10.7 | 0.0 | 4.1 |
|  | 2-methyl-2-butene | 1.75E+09 | 6.0 | 1.8 | - | - | 9.6 | 0.0 | 21.7 | 0.0 | 24.1 |
|  | 2-methyl-2-pentene | 1.25E+09 | 2.1 | 0.6 | 0.6 | 0.6 | - | - | - | - | - |
|  | 2-methylheptane | 1.26E+08 | 7.1 | 2.1 | - | - | 8.9 | 0.0 | 5.6 | 0.0 | 8.6 |
|  | 2-methylhexane | 3.52E+08 | 6.7 | 2.0 | - | - | 30.2 | 0.0 | 8.5 | 0.0 | 20.2 |
|  | 2-methylpentane | 9.74E+08 | 50.2 | 15.1 | 1.0 | 1.0 | - | 2.1 | 11.5 | 0.0 | 17.0 |
|  | 2,2-dimethylbutane | 1.46E+09 | 1.5 | 0.5 | - | - | 64.8 | 0.0 | 19.8 | 0.0 | 0.0 |
|  | 2,3-dimethyl-2-pentene | 2.40E+08 | 7.5 | 2.3 | 1.0 | 1.0 | 14.7 | 0.0 | 5.2 | 0.0 | 14.4 |
|  | 2,3-dimethylbutane | 1.08E+09 | 2.8 | 0.8 | 2.0 | 2.0 | 54.1 | 4.5 | 16.1 | 0.0 | 58.9 |
|  | 2,3,4-trimethylpentane | 1.64E+08 | 5.3 | 1.6 | - | - | 8.5 | 0.0 | 27.8 | 0.0 | 23.8 |
|  | 2,4-dimethylpentane | 3.52E+08 | - | - | - | - | 2.6 | 0.0 | 19.2 | 0.0 | 18.9 |
|  | 3-ethyltoluene | 1.87E+07 | 15.8 | 4.7 | - | - | 9.1 | 0.5 | 34.4 | 0.0 | 18.9 |
|  | 3-methyl-1-butene | 2.29E+09 | 29.5 | 8.9 | - | - | - | - | - | - | - |
|  | 3-methylheptane | 1.20E+08 | 5.7 | 1.7 | 2.9 | 2.9 | - | 0.8 | 6.0 | 0.0 | 4.3 |
|  | 3-methylhexane | 3.29E+08 | 24.5 | 7.4 | - | - | 2.6 | 0.9 | 9.6 | 0.0 | 15.7 |
|  | 3-methylpentane | 8.73E+08 | 12.5 | 3.8 | - | - | 4.7 | 0.0 | 7.4 | 0.0 | 23.7 |
|  | 4-ethyltoluene | 1.89E+07 | 7.7 | 2.3 | 3.1 | 3.1 | 27.2 | 0.0 | 66.1 | 0.0 | 65.9 |
|  | 4-methyl-1-pentene | 1.25E+09 | 27.2 | 8.2 | 0.7 | 0.7 | - | - | - | - | - |
|  | 4-methylheptane | 1.25E+08 | 5.6 | 1.7 | 1.8 | 1.8 | - | 0.0 | 9.4 | 0.0 | 6.9 |
|  | alpha-pinene | 3.47E+07 | 6.2 | 1.9 | - | - | 17.4 | 0.9 | 87.9 | 0.0 | 60.7 |
|  | acetylene | 5.05E+10 | 2858.9 | 857.7 | 9.2 | 9.2 | 861.8 | 37.4 | 859.9 | 12.0 | 834.1 |
|  | benzene | 4.61E+08 | 232.0 | 69.6 | 72.4 | 72.4 | 282.1 | 4.8 | 126.3 | 0.8 | 217.8 |
|  | butane | 6.18E+09 | 24.8 | 7.4 | 29.2 | 29.2 | 40.2 | 0.0 | 258.0 | 1.5 | 282.4 |
|  | butylbenzene | 7.35E+06 | 8.5 | 2.6 | - | - | 5.2 | 0.5 | 111.3 | 0.0 | 13.1 |
|  | c-1,3-dimethylcyclopentane | 3.52E+08 | - | - | - | - | 0.7 | 0.0 | 12.1 | 0.0 | 2.2 |
|  | c-2-butene | 3.50E+09 | 11.7 | 3.5 | 0.9 | 0.9 | 15.2 | 0.6 | 87.6 | 1.1 | 60.9 |
|  | c-2-hexene | 1.25E+09 | 6.1 | 1.8 | 14.4 | 14.4 | 18.3 | 0.0 | 17.3 | 0.0 | 28.3 |
|  | c-2-pentene | 2.29E+09 | 8.4 | 2.5 | - | - | 61.5 | 0.0 | 68.2 | 0.0 | 34.9 |
|  | c-3-hexene | 1.25E+09 | 7.2 | 2.2 | - | - | - | - | - | - | - |
|  | cyclohexane | 4.32E+08 | 51.9 | 15.6 | - | - | 1.5 | 0.0 | 58.7 | 0.0 | 3.6 |
|  | cyclohexene | 3.87E+08 | 14.5 | 4.4 | 3.7 | 3.7 | 4.5 | 0.0 | 4.4 | 0.0 | 14.0 |
|  | cyclopentane | 1.19E+09 | 12.6 | 3.8 | 1.8 | 1.8 | 27.3 | 0.0 | 34.7 | 0.0 | 14.6 |
|  | cyclopentene | 3.56E+08 | 95.5 | 28.7 | - | - | 2.0 | 0.0 | 23.9 | 0.0 | 12.4 |
|  | cyclopropane | 1.23E+10 | 2.9 | 0.9 | - | - | - | - | - | - | - |
|  | decane | 8.42E+06 | 2.5 | 0.8 | 33.4 | 33.4 | 5.5 | 9.4 | 177.6 | 0.0 | 178.5 |
|  | dodecane | 1.04E+06 | 108.3 | 32.5 | 16.1 | 16.1 | - | - | - | - | - |
|  | ethane | 6.13E+10 | 115.5 | 34.7 | 83.3 | 83.3 | 154.4 | 27.4 | 147.3 | 0.0 | 122.3 |
|  | ethene | 5.90E+10 | 77.3 | 23.2 | 28.1 | 28.1 | 2958.1 | 51.2 | 1414.3 | 9.6 | 2302.9 |
|  | ethylbenzene | 6.28E+07 | 3.9 | 1.2 | 1.0 | 1.0 | 24.9 | 0.0 | 60.7 | 0.0 | 58.1 |
|  | heptane | 2.43E+08 | 5.9 | 1.8 | - | - | 137.1 | 0.0 | 16.7 | 0.0 | 87.9 |
|  | hexane | 6.10E+08 | 15.4 | 4.6 | 2.4 | 2.4 | 239.4 | 69.1 | 27.6 | 0.0 | 115.7 |
|  | hexylbenzene | 1.00E+07 | 16.6 | 5.0 | - | - | - | - | - | - | - |
|  | i-butane | 8.15E+09 | 42.7 | 12.8 | 42.2 | 42.2 | 5.0 | 0.4 | 91.0 | 0.0 | 0.0 |
|  | i-butene | 3.47E+09 | 71.7 | 21.5 | 5.5 | 5.5 | 123.7 | 0.0 | 525.4 | 0.0 | 328.2 |
|  | i-pentane | 1.38E+09 | 34.0 | 10.2 | 29.9 | 29.9 | 0.0 | 0.0 | 0.0 | 0.0 | 208.9 |
|  | isoprene | 1.05E+09 | 56.0 | 16.8 | - | - | 84.9 | 0.0 | 39.6 | 0.0 | 3.9 |
|  | i-propylbenzene | 2.92E+07 | 4.8 | 1.4 | 0.8 | 0.8 | 8.6 | 0.0 | 105.2 | 0.0 | 69.9 |
|  | limonene | 1.05E+07 | 7.9 | 2.4 | - | - | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | m-xylene | 4.70E+07 | 26.4 | 7.9 | 1.1 | 1.1 | 38.8 | 0.0 | 56.9 | 0.0 | 5.6 |
|  | methylcyclohexane | 5.00E+08 | 14.4 | 4.3 | - | - | 5.2 | 0.0 | 5.9 | 0.0 | 12.3 |
|  | methylcyclopentane | 6.17E+08 | 11.2 | 3.4 | - | - | 2.7 | 0.0 | 12.1 | 0.0 | 11.0 |
|  | naphthalene | 1.52E+06 | 45.9 | 13.8 | 1.6 | 1.6 | - | - | - | - | - |
|  | nonane | 2.63E+07 | 36.1 | 10.8 | - | - | 116.1 | 0.0 | 3.5 | 0.0 | 93.6 |
|  | o-xylene | 3.74E+07 | 5.2 | 1.6 | - | - | 24.8 | 0.0 | 68.1 | 0.0 | 62.4 |
|  | octane | 8.18E+07 | 7.5 | 2.3 | 0.9 | 0.9 | 15.4 | 0.0 | 3.5 | 0.0 | 16.7 |
|  | p-xylene | 5.65E+07 | 4.8 | 1.4 | 3.8 | 3.8 | 20.3 | 1.0 | 45.6 | 0.0 | 73.9 |
|  | pentane | 1.97E+09 | 12.0 | 3.6 | 15.6 | 15.6 | 12.7 | 0.0 | 62.2 | 0.0 | 3.5 |
|  | propane | 1.70E+10 | 37.4 | 11.2 | 32.6 | 32.6 | 31.3 | 0.0 | 15.8 | 0.0 | 10.7 |
|  | propene | 1.95E+10 | 696.2 | 208.9 | 6.3 | 6.3 | 1123.0 | 5.4 | 1149.0 | 14.3 | 1192.6 |
|  | propylbenzene | 2.16E+07 | 16.6 | 5.0 | 1.4 | 1.4 | 15.1 | 0.0 | 36.2 | 0.0 | 29.6 |
|  | propyne | 5.04E+09 | 72.3 | 21.7 | - | - | 86.7 | 0.8 | 100.1 | 0.1 | 95.2 |
|  | sec-butylbenzene | 1.29E+07 | 39.4 | 11.8 | 1.6 | 1.6 | - | - | - | - | - |
|  | styrene | 3.51E+07 | 8.2 | 2.5 | - | - | 12.8 | 0.0 | 7.7 | 0.0 | 18.7 |
|  | tetradecane | 9.94E+04 | 4.9 | 1.5 | 0.9 | 0.9 | - | - | - | - | - |
|  | toluene | 1.58E+08 | 84.7 | 25.4 | 3.0 | 3.0 | 112.0 | 1.6 | 34.8 | 0.3 | 76.1 |
|  | 1,3-hexadiene (trans) | 1.25E+09 | 6.3 | 1.9 | - | - | 7.8 | 0.0 | 30.5 | 0.0 | 7.0 |
|  | t-2-butene | 2.75E+09 | 61.0 | 18.3 | 4.3 | 4.3 | 54.7 | 1.1 | 119.8 | 0.4 | 83.5 |
|  | t-2-hexene | 1.25E+09 | 9.5 | 2.9 | - | - | 10.2 | 0.0 | 13.4 | 0.0 | 10.4 |
|  | t-2-pentene | 2.29E+09 | 15.7 | 4.7 | - | - | 105.4 | 0.0 | 28.8 | 0.0 | 106.8 |
|  | tridecane | 3.29E+05 | 47.4 | 14.2 | 1.9 | 1.9 | - | - | - | - | - |
|  | undecane | 3.26E+06 | 93.7 | 28.1 | 15.8 | 15.8 | 2.3 | 2.6 | 46.6 | 0.0 | 76.5 |
| SVOC | C* = 10-2 µg m-3 | 1.00E-02 | 4.8 | 1.7 | 3.2 | 2.1 | 31.0 | 0.3 | 21.8 | 3.8 | 17.6 |
|  | C* = 10-1 µg m-3 | 1.00E-01 | 4.8 | 2.8 | 4.4 | 3.1 | 48.6 | 0.5 | 38.1 | 6.1 | 27.6 |
|  | C* = 100 µg m-3 | 1.00E+00 | 6.4 | 3.4 | 4.7 | 3.8 | 24.7 | 0.2 | 28.7 | 7.7 | 18.7 |
|  | C* = 101 µg m-3 | 1.00E+01 | 4.8 | 10.6 | 7.0 | 4.5 | 61.8 | 0.6 | 76.6 | 14.0 | 56.4 |
|  | C* = 102 µg m-3 | 1.00E+02 | 4.8 | 23.5 | 7.0 | 4.9 | 85.5 | 0.8 | 118.8 | 3.1 | 73.6 |
|  | C* = 103 µg m-3 | 1.00E+03 | 11.2 | 158.8 | 16.4 | 13.4 | 15.0 | 0.1 | 1.8 | 0.0 | 8.0 |
| IVOC | C* = 104 µg m-3 | 1.00E+04 | 25.6 | 285.1 | 10.4 | 8.4 | 56.2 | 0.5 | 9.7 | 0.0 | 26.6 |
|  | C* = 105 µg m-3 | 1.00E+05 | 80.0 | 34.1 | 4.0 | 5.9 | 984.0 | 9.4 | 196.3 | 12.6 | 493.3 |
|  | C* = 106 µg m-3 | 1.00E+06 | 1459.4 | 39.1 | 20.0 | 11.3 | 4901.3 | 46.6 | 3613.6 | 12.4 | 3814.1 |
|  | C* = 107 µg m-3 | 1.00E+07 | 1459.4 | 39.1 | 20.0 | 11.3 | 4901.3 | 46.6 | 3613.6 | 12.4 | 3814.1 |

Table 3: VBS yields for S/IVOC for non-idle and idle emissions.

| Ground Idle Yields |  |  |  |  | Non-Ground Idle Yields |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| a1 | b1 | c1 | d1 |  | a1 | b1 | c1 | d1 |
| 0.00 | 0.10 | 0.10 | 0.20 |  | 0.05 | 0.10 | 0.00 | 0.00 |

8 List of figures

Figure 1: Schematics that illustrate the SOA mechanism for the (a) T-SOA model, (b) Robinson-2007 method and (c) Hybrid method.

Figure 2: Average of the measured elemental carbon and POA emissions and sulfate and SOA formation from aircraft exhaust (Miracolo et al., 2011;Miracolo et al., 2012). CFM56 and T63 are gas turbine engines. JP8 is a petroleum-based aviation fuel, FT is a Fischer-Tropsch fuel derived from coal and Blend is a 50:50 JP8:FT mixture. The results for CFM56-JP8-Ground Idle are the average of three separate experiments and the results for T63-FT-Ground Idle are the average of two separate experiments. We did not perform a cruise experiment for T63-Blend.

Figure 3: Emission factors for S/IVOC and VOC (SOA precursors) and measured SOA production. The results for CFM56-JP8-Ground Idle are the average of three separate experiments and the results for T63-FT-Ground Idle are the average of two separate experiments. We did not perform a cruise experiment for T63-Blend.

Figure 4: Estimated OH exposure range (molecules hr cm-3) for the twelve different experiments. The median OH exposure is shown by the orange cross and the standard error of the mean is shown using green bars. We use the median value in our analysis.

Figure 5: Scatter plots of predicted versus measured OA mass for the T-SOA model and two versions of the NT-SOA model (Robinson-2007 and Hybrid). The top row shows experiments performed with the CFM56 engine and the bottom row shows experiments performed with the T63 engine.

Figure 6: Estimated NT-SOA yield plotted as a function of COA. For reference, we also include SOA yields for n-dodecane and n-tridecane (dotted grey lines) from Presto et al. (2010).

Figure 7: Model predictions of OA compared to those measured during each experiment. The two sets of symbols for the measured values show the range of uncertainty, as discussed in the text. NT-SOA is predicted using the Hybrid method using best fits for each experiment. For some experiments, OH concentrations are calculated using intermittent measurements which results in step changes in the OH exposure. For those experiments, we see sudden changes in SOA formation with OH exposure.

Figure 8: Measured OA compared to model predictions using best-fits of the Robinson-2007 method for the T63-Blend-Ground Idle experiment. The two sets of symbols for the measured values show the range of uncertainty, as discussed in the text.

Figure 9: Estimated SOA yield plotted as a function of COA for different S/IVOC volatility bins that contribute more than 15% of the predicted NT-SOA mass (symbols). For reference, we also plot SOA yields for n-decane (estimated), n-dodecane, n-tetradecane, n-hexadecane and n-octadecane (estimated) (dotted lines) from Presto et al. (2010). The colors connect the estimate NT-SOA yields for each volatility bin (symbols) to the n-alkane with same volatility (dotted lines). For example, the SOA yields for the C*=106 µg m-3 bin for all the experiments are plotted with blue squares and the SOA yield for C* equivalent n-dodecane (C12) is plotted with a blue dotted line.

9 Supplementary material

Table S.1: SAPRC lumping and reaction rate for each speciated precursor

| Species | Lumping | kOH (cm3 molecules-1 s-1) |
| --- | --- | --- |
| 1-butene | OLE1 | 3.14E-11 |
| 1-heptene | OLE1 | 3.34E-12 |
| 1-hexene | OLE1 | 3.70E-11 |
| 1-methylcyclohexene | OLE1 | 9.64E-12 |
| 1-octene | OLE1 | 3.70E-11 |
| 1-pentene | OLE1 | 3.14E-11 |
| 1;2-butadiene | OLE1 | 2.60E-11 |
| 1;2-diethylbenzene | ARO1 | 5.80E-12 |
| 1;2;3-trimethylbenzene | ARO2 | 3.27E-11 |
| 1;2;4-trimethylbenzene | ARO2 | 3.25E-11 |
| 1;2;4;5-tetramethylbenzene | ARO2 | 3.25E-11 |
| 1;3-butadiene | OLE2 | 6.66E-11 |
| 1;3-diethylbenzene | ARO2 | 3.25E-11 |
| 1;3;5-trimethylbenzene | ARO2 | 5.67E-11 |
| 1;4-diethylbenzene | ARO2 | 3.25E-11 |
| 2-ethyltoluene | ARO1 | 1.18E-11 |
| 2-methyl-1-butene | OLE2 | 6.10E-11 |
| 2-methyl-1-pentene | OLE2 | 6.30E-11 |
| 2-methyl-2-butene | OLE2 | 8.69E-11 |
| 2-methyl-2-pentene | OLE2 | 8.90E-11 |
| 2-methylheptane | ALK4 | 4.77E-12 |
| 2-methylhexane | ALK4 | 4.77E-12 |
| 2-methylpentane | ALK4 | 5.20E-12 |
| 2;2-dimethylbutane | NONE | 2.23E-12 |
| 2;3-dimethyl-2-pentene | OLE2 | 1.03E-10 |
| 2;3-dimethylbutane | ALK4 | 5.78E-12 |
| 2;3;4-trimethylpentane | ALK4 | 6.60E-12 |
| 2;4-dimethylpentane | ALK4 | 6.75E-12 |
| 3-ethyltoluene | ARO1 | 1.19E-11 |
| 3-methyl-1-butene | OLE1 | 3.18E-11 |
| 3-methylheptane | ALK4 | 5.20E-12 |
| 3-methylhexane | ALK5 | 8.11E-12 |
| 3-methylpentane | ALK4 | 5.20E-12 |
| 4-ethyltoluene | ARO2 | 1.86E-11 |
| 4-methyl-1-pentene | OLE2 | 6.30E-11 |
| 4-methylheptane | ALK4 | 5.20E-12 |
| alpha pinene | TERP | 5.23E-11 |
| acetylene | NONE | 8.15E-13 |
| benzene | ARO1 | 1.22E-12 |
| butane | NONE | 2.36E-12 |
| butylbenzene | ARO1 | 4.50E-12 |
| c-1;3-dimethylcyclopentane | ALK5 | 9.64E-12 |
| c-2-butene | OLE2 | 5.64E-11 |
| c-2-hexene | OLE1 | 3.70E-11 |
| c-2-pentene | OLE2 | 6.50E-11 |
| c-3-hexene | OLE1 | 3.70E-11 |
| cyclohexane | ALK5 | 6.97E-12 |
| cyclohexene | OLE2 | 6.77E-11 |
| cyclopentane | ALK4 | 4.97E-12 |
| cyclopentene | OLE2 | 6.70E-11 |
| cyclopropane | NONE | 8.15E-14 |
| decane | ALK5 | 1.10E-11 |
| dodecane | ALK5 | 1.32E-11 |
| ethane | NONE | 2.48E-13 |
| ethene | NONE | 8.52E-12 |
| ethylbenzene | ARO1 | 7.00E-12 |
| heptane | ALK4 | 6.76E-12 |
| hexane | ALK4 | 5.20E-12 |
| hexylbenzene | ARO2 | 1.13E-10 |
| i-butane | NONE | 2.36E-12 |
| i-butene | OLE1 | 3.14E-11 |
| i-pentane | ALK4 | 3.80E-12 |
| isoprene | ISOP | 1.01E-10 |
| i-propylbenzene | ARO1 | 6.30E-12 |
| limonene | TERP | 1.64E-10 |
| m-xylene | ARO2 | 2.31E-11 |
| methylcyclohexane | ALK5 | 9.64E-12 |
| methylcyclopentane | ALK4 | 3.80E-12 |
| naphthalene | ARO2 | 2.30E-11 |
| nonane | ALK5 | 9.70E-12 |
| o-xylene | ARO2 | 1.36E-11 |
| octane | ALK5 | 8.11E-12 |
| p-xylene | ARO2 | 1.43E-11 |
| pentane | ALK4 | 3.80E-12 |
| propane | NONE | 1.09E-12 |
| propene | NONE | 2.63E-11 |
| propylbenzene | ARO1 | 5.80E-12 |
| propyne | NONE | 7.13E-12 |
| sec-butylbenzene | ARO1 | 5.80E-12 |
| styrene | ARO2 | 5.80E-11 |
| tetradecane | ALK5 | 1.79E-11 |
| toluene | ARO1 | 5.63E-12 |
| 1;3-hexadiene (trans) | OLE2 | 1.12E-10 |
| t-2-butene | OLE2 | 6.40E-11 |
| t-2-hexene | OLE1 | 3.70E-11 |
| t-2-pentene | OLE2 | 6.70E-11 |
| tridecane | ALK5 | 1.51E-11 |
| undecane | ALK5 | 1.23E-11 |

Table S.2: VBS yields for SAPRC lumped species (Murphy and Pandis, 2010)

| Group | High NOx (Low Yield) |  |  |  |  | Low NOx (High Yield) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | C* (µg/m3) |  |  |  |  | C* (µg/m3) |  |  |  |
|  | 1 | 10 | 100 | 1000 |  | 1 | 10 | 100 | 1000 |
| ALK4 | 0.0000 | 0.0375 | 0.0000 | 0.0000 |  | 0.0000 | 0.0750 | 0.0000 | 0.0000 |
| ALK5 | 0.0000 | 0.1500 | 0.0000 | 0.0000 |  | 0.0000 | 0.3000 | 0.0000 | 0.0000 |
| OLE1 | 0.0008 | 0.0045 | 0.0375 | 0.1500 |  | 0.0045 | 0.0090 | 0.0600 | 0.2250 |
| OLE2 | 0.0030 | 0.0255 | 0.0825 | 0.2700 |  | 0.0225 | 0.0435 | 0.1290 | 0.3750 |
| ARO1 | 0.0107 | 0.2571 | 0.4821 | 0.7500 |  | 0.0107 | 0.2571 | 0.7500 | 0.9643 |
| ARO2 | 0.0015 | 0.1950 | 0.3000 | 0.4350 |  | 0.0750 | 0.3000 | 0.3750 | 0.5250 |
| ISOP | 0.0003 | 0.0225 | 0.0150 | 0.0000 |  | 0.0090 | 0.0300 | 0.0150 | 0.0000 |
| SESQ | 0.0750 | 0.1500 | 0.7500 | 0.9000 |  | 0.0750 | 0.1500 | 0.7500 | 0.9000 |
| TERP | 0.0120 | 0.1215 | 0.2010 | 0.5070 |  | 0.1073 | 0.0918 | 0.3587 | 0.6075 |