Interactive comment on “Brown carbon’s emission factors and optical characteristics in household biomass burning: Developing a novel algorithm for estimating the contribution of brown carbon” by Jianzhong Sun et al.

J. Sun et al.

zhxi.2006@163.com

Reply to Referee 1

General comment:

This is a manuscript that reports on the emission factors and optical characteristics of BB-derived BrC and development of a novel algorithm for estimating the contribution of BrC. The results indicated the mean emission factors of BB-BrC are 0.71 g/kg, which were affected by the plant type and burning styles. The average AAE value was 2.46 ± 0.53, which are much higher than that of coal-chunks combustion smoke. The contribution of absorption by BB-BrC to the total absorption by BC + BrC were also calculated, is 50.8%. Finally, a novel algorithm was developed for estimating the FBrC for any combustion sources. This is an interesting research about the emission factors and light-absorption characteristics of BrC emitted from biomass burning. I think the manuscript can be accepted after the following comments are addressed.

Response:

Thanks a lot for the positive comment. The recommendation for publication in ACP is encouraging. We would further improve our manuscript according to the comments and suggestions below.

Comments 1:

Line 11: what’s the meaning of “0.24, 2.18”?

Response:

Thanks. When we initially submitted our manuscript to editorial office, the editor suggested us not to use arithmetic mean but other forms, such as the median value and/or a range because the values of EFBrC (or EFBC) for different samples differed by more than an order of magnitude. Following the suggestion, we turned to use geomeans instead of arithmetic means. Line 11 in original version described the geomean (i.e., 0.71g/kg) calculated for EFBrC and the (lower, upper) limits (i.e., 0.24, 2.18) calculated via a geomean (i.e., 0.71g/kg) divided/multiplied by the geometric standard deviation (i.e., 2.95). We added a note to Table 1 to show how the geomean and range (lower and upper limits) were obtained. In addition, we changed the upper limit of 2.18 in original version to 2.09 in revised version to correct for wrong calculation in original version.

Comments 2:

Lines 70-76: several important references for the BrC from biomass burning in China were missed, such as Fan et al. (2016) ACP, 16, 13321-13340; Huo et al. (2018) Atmos. Environ., 191, 490-499, etc.

Response:

Thanks for this reminder. We added these citations proposed by the reviewer (Revised version, lines 72-73).

Comments 3:

Experimental section: accuracy, precision, and repeatability are not well quantified or discussed in this paper. The 11 biomass fuels are each burned and sampled once. The filter sample for each fire is collected in background air, so ambient aerosol may present in the sample. These may be reasonable experimental procedures, but the following information is missing: i) Blank filter sample for ambient air only to determine the background concentrations; ii) Repetitions of identical sample burns to determine the repeatability of the fires and the analysis procedure.

Response:

So sorry that we didn’t completely and clearly describe the accuracy, precision, and repeatability. In the revised version, we added the missing information and bettered the unclear description, particularly on the two key concerns: i) Blank filter sample for ambient air only to determine the background concentrations; ii) Repetitions of identical sample burns to determine the repeatability of the fires and the analysis procedure.

The revised version is as follows: “Each of the 11 biomass fuels was burned for 2-3 times and each time was with a quartz filter for particle collection. The 2-3 duplicate samples helped check the reproducibility and analysis procedure. Background concentrations of ambient air were determined with blank filters.” (lines 107-109, revised version).

Comments 4:

Please reduce the number of significant digits (2-3 is preferred) in Table 1, S1, and possibly in the main text. For example, “7.259” (four significant digits) can be present as “7.26” (maximum three significant digits). Please double check such errors throughout the entire manuscript.

Response:

Thanks for this reminder. We checked throughout the whole manuscript and reduced the number of significant digits after the decimal point to 2 in Table 1 and Table S1 except for EFBrC and EFBC of the rape straw. We actually collected 2 samples for rape straw. The experimental results of the duplicates were extremely close, which made the standard deviations be 0.002 for EFBrC and 0.001 for EFBC. When we initially submitted our manuscript to editorial office, the editor suggested us to increase the number of significant digit after the decimal point from 2 (0.00) to 3 (0.002 and 0.001 specifically for the EFBrC and EFBC of rape straw sample) to avoid the uncertainty value of 0.00. For this reason, we maintained the two data of rape straw with 3 significant digits after the decimal point, as 7.259±0.002 and 2.537±0.001 (See revised version, Table 1 and S1), while the data of other biomass fuels were designated with 2 significant digits after the decimal point.

Comments 5:

Lines 205-208: The ratios of EFBrC to EFBC for different samples were varied with very large range (the highest one is 10.0 and the lowest one is 1.5). Why? Please add some explanation. This is very important for the estimation of the contribution of BB BrC.

Response:

Thanks for this suggestion. Sure the ratios of EFBrC to EFBC for different samples varied with very large ranges; for example the highest one is 10.0 and the lowest one is 1.5. Although the reasons for the large range in RBrC/BC ratio among different biomass cases involves very complicated factors, they are essentially attributed to the differences in chemical composition and physical structure. It is acknowledged that both BrC and BC are products of incomplete combustion of biomass fuels (Andreae and Gelencsér, 2006; Yan et al., 2015). Different biomass fuels are composed by different organics that have different combustion performances (Reid et al., 2005; Saleh et al., 2014); meanwhile, different biomass fuels are also different in densities and moistures (Shen et al., 2014; Jacobson et al., 2015), which are also potential to exert influences on the combustion performance. The combustion performance relates to something like the combustion speed and temperature, both of which are important to the formation of BrC and BC. Usually a low combustion temperature is more favorable for BrC formation and a relatively high combustion temperature is more favorable for BC formation (Chen and Bond, 2010; Bond et al., 2013; Shen et al., 2014). This suggests that the generation processes of BC and BrC are often not synchronous but in opposite trend, which made the values of RBrC/BC vary terribly.

We understand the importance of the RBrC/BC for the estimation of the contribution of biomass BrC and accordingly added some explanations in lines 223-237 in our revised version.

Comments 6:

Figure 2: the data of BrC from BB and coal combustion should be labelled with different markers.

Response:

Thank you. In our revised version, the data of BrC from BB and from coal combustion in Fig. 2 have been labelled with red and blue markers, respectively.

Comments 7:

Lines 252-258: China’s BrC and BC emissions from biomass fuels burned in household stoves were calculated. This section is associated with high uncertainties due to the reliable consumption amounts of different types of biomass fuels and forms, representative BrC emission factors from this study. I’d like to suggest to add discussions on uncertainties and limitations.

Response:

Thanks for this suggestion. Lines 252-258 (previous version) described China’s BrC and BC emissions from biomass fuels burned in household stoves. The calculated emissions indeed contained uncertainties resulting from the consumption amounts and forms of different types of biomass fuels as well as the representativity of BrC emission factors measured in this study. We added discussions on the uncertainties and limitations (lines 287-291, revised version).

Comments 8:

Section 3.4, Lines 295-306: To construct the function for FBrC, with AAE as the independent variable, four pairs of FBrC vs AAE values were investigated: one pure BC and three pure BrC. For the three pure BrC, I have two questions: 1) why the average values of FBrC vs AAE rather than the data of each sample were used to construct the function between FBrC and AAE? 2) As shown in Table S3, the AAE values of WSOC or MSOC in the literature were determined in solution. However the AAE values of BrC were determined with the integrating sphere method in this paper and the previous study (Sun et al., 2017). How about the differences of AAE values measured with these two methods. You should add some discussions to interpret that.

Response:

Thanks for this question and suggestion.

(A) We described how we constructed the relation in lines 331-341 (revised version). That is, to construct the function for FBrC, with AAE as the independent variable, we managed to gather four pairs of FBrC vs AAE values. Two of these pairs were theoretically for pure BC and pure BrC, respectively. For pure BC (free of BrC), the AAE and FBrC were 1.0 (Lack and Langridge, 2013; Laskin et al., 2015; Yan et al., 2015; Zhang et al., 2020) and 0.0, respectively, and for samples of pure BrC (free of BC), we averaged over the AAE values in the literature for WSOC or MSOC, thus obtaining an AAE value of 6.09 ± 1.45 (Hoffer et al., 2006; Hecobian et al., 2010; Voisin et al., 2012; Srinivas and Sarin, 2013, 2014; Srinivas et al., 2016; Lei et al., 2018) (Part I in Table S3). The other two pairs of the FBrC vs AAE values were obtained from our previous and current measurements. The previous study (Sun et al., 2017) demonstrated that, when AAE was 1.58, FBrC was 0.265. In the present study, as mentioned in Section 3.3, an AAE of 2.46 led to an FBrC of 0.508. These four FBrC vs AAE pairs were used to construct the relationship between FBrC and AAE (Figure 5).

(B) The question why the average values of FBrC vs AAE rather than the data of each sample were used to construct the function between FBrC and AAE is worth explaining. The same question had actually been raised by the editor and we had explained the reason in advance. On the one hand, we know, each of the latter three points (i.e., 1.58, 0.265; 2.46, 0.508; 6.09, 1.00) in Figure 5 is the average of a number of data, and therefore each of them can be potentially replaced with a cluster of individual dots if we like; yet on the other hand, the first point (0.00,1.00) is not originated from averaging over a cluster of individuals but from theoretical consideration, and thus there are no cluster of individual dots usable to replace this single point. Under the circumstances, replacing each of the latter three points with an individual cluster of dots while leaving the first point with single dot will substantially lower the weight of the first point from 25% to almost being negligible. Given the theoretical significance of the first point, this is not only unfair but also unacceptable. For this consideration, we preferred to the average value for each of the latter three points so that all the four points in Figure 5 were put weights equally (25%). Additionally, compared with a cluster of individuals, an average is usually closer to or more representative of the true value and hence is more persuasive. We added an explanation in our revised version (lines 341-343).

(C) As regards the need to add some discussion on the differences between AAE values measured with IS method and the AAE values measured through WSOC or MSOC, the former is for the entirety of a sample including BrC+BC whereas the latter is for BrC alone (free of BC).

Comments 9:

Table S1: The abbreviation of “M%, CR, FW, PF” should be illustrated in full name.

Response:

Thanks for reminder. We gave the full names of the abbreviations as a note to Table S1, as follows:

Note: M% - moisture on air-dry basis (%); 11 biomass fuels used in this study were divided into 3 categories: CR - crop residue; FW - fire wood; PF - pellet fuel.

Comments 10:

Is Fig S4 cited from the literature of authors (Sun, J., Zhi, G., et al., Emission factors and light absorption properties of brown carbon from household coal combustion in China, Atmos. Chem. Phys., 17, 4769-4780)? If so, please add references in the caption.

Response:

Thanks for reminder. We added‘Sun et al., 2017’in Fig. S4 accordingly.

References:

Andreae, M. O. and A. Gelencsér.: Black carbon or brown carbon? The nature of light-absorbing carbonaceous aerosols, Atmo. Chem. Phys., 6: 3131-3148, doi: 10.5194/acp-6-3131-2006, 2006.

Bond, T., Doherty, S., Fahey, D., Forster, P., Berntsen, T., DeAngelo, B., Flanner, M., Ghan, S., Kärcher, S., Koch, D., Kinne, S., Kondo, Y., Quinn, P., Sarofim, M., Schultz, M., Schulz, M., Venkataraman, C., Zhang, H., Zhang, S., Bellouin, N., Guttikunda, S., Hopke, P., Jacobson, M., Kaiser, J., Klimont, Z., Lohmann, U., Schwarz, J., Shindell, D., Storelvmo, T., Warren, S., and Zender, C.: Bounding the role of black carbon in the climate system: A scientific assessment, Journal of Geophysical Research: Atmospheres, 118(11): 5380-5552, DOI: 10.1002/jgrd.50171, 2013.

Chen, Y. and Bond, T. C.: Light absorption by organic carbon from wood combustion, Atmos. Chem. Phys., 2010, 10, 1773–1787, doi:10.5194/acp-10-1773-2010, 2010.

Hecobian, A., Zhang, X., Zheng, M., Frank, N., Edgerton, E. S., and Weber, R. J.: Water-soluble organic aerosol material and the light-absorption characteristics of aqueous extracts measured over the Southeastern United States, Atmos. Chem. Phys., 10(13), 5965-5977, doi: 10.5194/acp-10-5965-2010, 2010.

Hoffer, A., Gelencser, A., Guyon, P., Kiss, G., and Schmid, O.: Optical properties of humic-like substances (HULIS) in biomass-burning aerosols, Atmos. Chem. Phys., 6, 3563-3570, doi:10.5194/acp-6-3563-2006, 2006.

Jacobson, M. Z.: Effects of biomass burning on climate, accounting for heat and moisture fluxes, black and brown carbon, and cloud absorption effects, J. Geophys Res-Atmos., 119 (14), 8980−9002, doi: 10.1002/2014JD021861 , 2015.

Lack, D. A. and Langridge, J. M.: On the attribution of black and brown carbon light absorption using the Ångström exponent, Atmos. Chem. Phys., 13(20), 10535-10543, 10543, doi:10.5194/acp-13-10535-2013, 2013.

Laskin, A., Laskin, J., and Nizkorodov, S. A.: Chemistry of atmospheric brown carbon, Chem. Rev., 115 (10), 4335-4382, doi:10.1021/cr5006167, 2015.

Lei, Y., Shen, Z., Zhang, T., Zhang, Q., Wang, Q., Sun, J., Gong, X., Cao, J., Xu, H., and Liu, S.: Optical source profiles of brown carbon in size-resolved particulate matter from typical domestic biofuel burning over Guanzhong Plain, China, Sci. Total Environ., s622–623, 244-251, doi: 10.1016/j.scitotenv.2017.11.353, 2018.

Reid, J., Eck, T., Christoper, S., Koppmann, R., Dubovik, O., Eleuterio, D., Holben, B., Reid, and Zhang, J.:A review of biomass burning emissions part III: intensive optical properties of biomass burning particles, Atmos. Chem. Phys. 5, 827-849. doi: 10.5194/acp-5-799-2005, 2005.

Saleh, R., Robinson, E. S., Tkacik, D. S., Ahern, A. T., Liu, S., Aiken, A. C., Sullivan, R. C., Presto, A. A., Dubey, M. K., Yokelson, R. J., Donahue, N. M., and Robinson, A. L.: Brownness of organics in aerosols from biomass burning linked to their black carbon content, Nature Geosci., 2014, 7(9), 647-650, doi:10.1038/ngeo2220, 2014.

Shen, G., Xue, M., Chen, Y.,  Yang, C., Li, W., Shen, H., Huang, Y., Zhang, Y., Chen, H., Zhu, Y., Wu, H., Ding, A., and Tao, S.: Comparison of carbonaceous particulate matter emission factors among different solid fuels burned in residential stoves. Atmo. Environ., 89, 337-345, doi: 10.1016/j.atmosenv.2014.

Srinivas, B. and Sarin, M. M.: Light absorbing organic aerosols (brown carbon) over the tropical Indian Ocean: Impact of biomass burning emissions, Environ. Res. Lett., 8(4), 1-7, doi: 10.1088/1748-9326/8/4/044042, 2013.

Srinivas, B. and Sarin, M. M.: Brown carbon in atmospheric outflow from the Indo-Gangetic Plain: Mass absorption efficiency and temporal variability. Atmos. Environ., 89, 835-843, doi:10.1016/j.atmosenv.2014.03.030, 2014.

Srinivas, B., Rastogi, N., Sarin, M. M., Singh, A., and Singh, D.: Mass absorption efficiency of light absorbing organic aerosols from source region of paddy-residue burning emissions in the Indo-Gangetic Plain. Atmos. Environ., 125, 360-370, doi:10.1016/j.atmosenv.2015.07.017 , 2016.

Sun, J., Zhi, G., Hitzenberger, R., Chen, Y., Tian, C., Zhang, Y., Feng, Y., Cheng, M., Zhang, Y., Cai, J., Chen, F., Qiu, Y., Jiang, Z., Li, J., Zhang, G., and Mo, Y.: Emission factors and light absorption properties of brown carbon from household coal combustion in China, Atmos. Chem. Phys., 17(7), 4769-4780, doi: 10.5194/acp-17-4769-2017, 2017.

Voisin, D., Jaffrezo, J. L., Houdier, S., Barret, M., Cozic, J., King, M. D., France, J. L., Reay, H. J., Grannas, A., Kos, G., Ariya, P. A., Beine, H. J., and Domine, F.: Carbonaceous species and humic like substances (HULIS) in Arctic snowpack during OASIS field campaign in Barrow, J. Geophys. Res., 117, 1-17, doi: 10.1029/2011jd016612, 2012.Yan, C., Zheng, M., Sullivan, A. P., Bosch, C., Desyaterik, Y., Andersson, A., Li, X., Guo, X., Zhou, T., Gustafsson, Ö., and Collett, J. L.: Chemical characteristics and light-absorbing property of water-soluble organic carbon in Beijing: Biomass burning contributions, Atmos. Environ., 121, 4-12, doi:10.1016/j.atmosenv.2015.05.005, 2015.

Zhang, Y., Shen, Z., Zhang, B., Sun, J., Zhang, L., Zhang, T., Xu, H., Bei, N., Tian, J., Wang, Q., and Cao, J.: Emission reduction effect on PM2.5, SO2 and NOx by using red mud as additive in clean coal briquetting, Atmospheric Environment, 223, doi: 10.1016/j.atmosenv.2019.117203, 2020.